P710/1, P720/1

GEOMETRICAL & MECHANICAL

DRAWING

GEOMETRICAL & BUILDING

DRAWING

Paper 1

AUGUST, 2019

3 hours

JINJA JOINT EXAMINATIONS BOARD

Uganda Advanced Certificate of Education

MOCK EXAMINATIONS – AUGUST, 2019

GEOMETRICAL & MECHANICAL

DRAWING.

GEOMETRICAL & BUILDING

DRAWING

Paper 1

3 hours

INSTRUCTIONS:

This question paper consists of eight questions in two sections I and II

- Answer any Five questions. Not more than three questions may be answered from any one section.

- Use the drawing paper size A2 provided.

- Use both sides of the paper provided if necessary.

- All dimensions are in millimeters unless otherwise stated.

- The given figures are not drawn to scale. The answers to all questions must be drawn to the given dimensions.

- All questions carry equal marks.

SECTION I:

PLANE GEOMETRY

1.	A left-hand 12mm square has a lead of 60mm and internal diameter of 88mm.

Draw 1 ½ turns of this spring.	(20 marks)

2.	Construct full size a plate cam to satisfy the following specifications of the follower 	which offset 20mm to right of the centre line.

0o – 150o	Simple Harmonic motion rise 40mm

150o – 210o 	Dwell

210o – 360o	Uniform acceleration and retarodation fall 40mm

Cam rotation – clockwise

The nearest approach of the follower centre to the horizontal centre line of the Cam is 	50mm and the cam shaft diameter is 25mm.	(20 marks)

3.	Figure 1 shows a mechanism, which consists of two cracks OA and BC and two links 	AC and DE.

AC = 135, OB = 120, OA = 25, AD = 80, BC = 40, DE = 60 and DP = 40

(i) 	Plot the locus D for one revolution of OA and	(12 marks)

(ii) 	the loas of P 			(8 marks)

4.	(a)	Figure 2 shows a roller with centre O of base circle radius 110mm. Circle B 		rolls along a curve with radius 110mm for one revolution from  X to Y.

(10 marks)

(b)	Then circle A also begins from point Q for half a revolution inside a base circle 		radius 90mm.		(6 marks)

(c)	Name the loci.		(4 marks)

SECTION II

5.	Figure 3 shows a bracket. Draw the bracket in isometric projection with X as the 	lowest part.

6.	Figure 4 shows two orthographic views of a casting drawn in first projection.

Draw full size:

(a)	the given views.

(b)	a first auxiliary plan on X1 – Y1

(c)	a second auxiliary elevation on X2 – Y2

7.	Figure 5 shows the vertical and horizontal traces of the oblique plane VTH in first 	angle projection.

From the true shape P1Q1R1 shown, draw the:

(a)	plan 			(9 marks)

(b)	front elevation		(7 marks)

(c)	true angle of inclination of the oblique plane to the horizontal plane.	(4m arks)

8.	Figure 6 shows two incomplete views of a right cone interpenetrated by a regular 	hexagonal prism.

(a)	copy the figure		(2 marks)

(b)	complete the views with curves of interpenetration.	(18 marks)