P720/2

GEOMETRICAL & BUILDING

DRAWING.

Paper 2

AUGUST, 2019

3 hours

JINJA JOINT EXAMINATIONS BOARD

Uganda Advanced Certificate of Education

MOCK EXAMINATIONS – AUGUST, 2019

GEOMETRICAL & MECHANICAL

DRAWING.

Paper 2

3 hours

INSTRUCTIONS TO CANDIDATES:

This question paper consists of two sections I and II a total of five questions.

- All questions are compulsory.

- All dimensions are in millimeters unless otherwise stated.

- A drawing paper size A2 is provided, use both sides if necessary.

- Where dimensions are not given, use your own discretion to determine missing dimensions.

- Figures in this paper are not drawn to scale.

- Interpret properly the specifications and figure provided before you attempt any question.

Figure 1 shows a line diagram plan of A RESIDENTIAL HOUSE drawn not to scale.

SPECIFICATIONS:

ROOF:	Hipped roof, the pitch is at 300 pitch angle covered with corrugated, galvanized iron sheets, on wooden purlins of 75mm x 50mm, on struts of 100mm x 50mm on rafters of 100mm x 50mm, on Tie beam 150mm x 50mm, on wall plate of 100mm x 75mm finished with Fascia boards boards of 250mmx 25mm.

WALL: 	Both internal and external wall are 230mm thick and 3000mm high to the underside of the wall plate built in English Bond.

FOUNDATION: Concrete strip foundation of 650mm wide and 230mm thick, 1200mm below the ground level.

FLOOR:	Finished with cement sand screed 25mm thick on concrete slab of 100mm thick, on hard core of 150mm thick, on well compacted marrum.

DOORS:   	D1	1500mm x 2100mm is double leaf metal casement with glass panes.

D2       1000mm x 2100mm is a flush doors

D3	1050mm x 2100mm framed ledged braced and battened door.

WINDOWS:	All windows are metallic casement windows with glass panes.

W1	1800mm x 1200mm

W3	 1000mm x 600mm

SPLASH APRON:	Concrete slab of 650mm wide x 150mm thick.

SECTION I

- Using the specifications given above, draw a proportional free hand isometric sketch of the building with corner A  in the fore ground.

(20 Marks)

SECTION II

- To a scale of 1:100, Draw; the ground plan with all symbols indicating Doors in their open positions, windows, hatches, fittings in the kitchen and toilets

(18 Marks)

- To a scale of 1:100, draw the vertical section on the cutting plane     X – X showing all the construction details.                                                		(22 Marks)

- To a scale of 1: 25 draw a cross section through a closed eave and show the following;

(i) Soffit board

(ii)Ceiling joist

(iii)Fascia board

(iv) Gutter 									(07 marks)

- At the bottom right hand corner of your drawing paper, print in the title block your; Name, Title of the drawing, Subject code and Date.		            (3 Marks)