735/3

GEOMETRICAL AND BUILDING DRAWING

Paper 3

AUGUST, 2019

3 HOURS

JINJA JOINT EXAMINATION BOARD

Uganda Certificate of Education

MOCK EXAMINATION – AUGUST, 2019

GEOMETRICAL AND BUILDING DRAWING

Paper 3

3 HOURS

INSTRUCTIONS TO CANDIDATES:

- This question paper consists of two sections I and II.

- Attempt ALL questions in this question paper.

- A drawing paper size A2 is provided, use both sides if necessary.

- Where dimensions are not given, use your discretion to determine missing dimensions.

- Figures in this paper are not drawn to scale.

- Interpret properly the specifications and figure provided before you attempt any question

Figure 1 provided below shows a ground plan of a” RESIDENTIAL HOUSE” not drawn to scale. The house is to be constructed using the following specifications:

SPECIFICATIONS:

ROOF:	Is a three gable ended house pitched at 300 pitch angle covered with corrugated, galvanized iron sheets, on wooden purlins of 75mm x 50mm, on struts of 100mm x 50mm on rafters of 100mm x 50mm, on Tie beam 150mm x 50mm, on wall plate of 100mm x 75mm finished with Fascia boards and verge boards

200mm x 25mm.

WALL: 	Both internal and external wall are 230mm thick and 3000mm high from the finished floor to the underside of the wall plate built in FLEMISH BOND,

FOUNDATION: Concrete strip foundation of 750mm wide and 250mm thick, 1200mm below the ground level.

FLOOR:	Finished with cement sand screed 25mm thick on site concrete 100mm thick, on hard core of 150mm thick, on well compacted marrum.

DOORS:  	D1	1000 x 2100mm is double leaf metal casement with glass panes.

D2	1500 x 2100mm is a framed ledged braced and battened door.

WINDOWS:	All windows are Louvered and fitted in wooden frames

W1	1800 x 1200mm

W3	  900 x 1200mm

SPLASH APRON:	Concrete slab of 650 by 150mm.

SECTION I

- Using the specifications given above, draw a proportional free hand isometric sketch of the building with corner P in the fore ground.                                  										(20Marks)

SECTION II

- a) To a scale of 1:100, Draw;

  - the ground plan with all symbols indicated	  	(20Marks)

  - the cross-section  X – X				            (25Marks)

3.    	To scale 1: 100, draw the front elevation from arrow S	  (10 marks)

4.	At the bottom right hand corner of your drawing paper, print in the 	title block your; Name, Index number, Title of the drawing, Subject 	code, Date.  	  							(05Marks)