OBUTU PASKWALE: SOROTI SECONDARY SCHOOL, PO BOX 174

AGRICULTURE NOTES

AGRICLTURAL MECHANISATION

DEF   This is the use of mechanical aid or machines to do farm jobs

REASONS [MERITS] FOR MECHANISING THE FARM

- Eases   work

- Saves time

- Economical for large scale production

- Increases the quality produce due to better uniformity

- Increases output by bringing more land into use

- Makes impossible task possible

- Reduces human drudgery

- Improves soil physical properties eg structure, porosity etc

- Increases output per person

- A more thorough job can be done than by hand

- It  solve the problems of laboure shortages

- Allows for better combination of enterprises

- Releases laboure to be use in farm operation that cannot be mechanized

- Help in proper planning of farm

FACTORS AGAINST MECHANISATION [DEMERITS]

- It’s expensive

- Skilled labour is required

- Causes unemployment

- Some jobs cannot be mechanized

- It cannot work well in hilly areas

- Exhaust smoke pollutes environment

- Machines may compact soil if use on wet soil

- Spare parts are expensive and scarce

- Not economical for small scale production

- High maintainces cost

- Clearing large land can lead to soil erosion

LIMITATIONS TO AGRICULTURAL MECHANISATION

- Land fragmentation

- Hand lab our is still considered cheaper than mechanization

- Its expensive to many farmers

- Poor marketing leading to low price resulting from over production

- Operation require skill man power

- Clearing large area of land encourages soil erosion

- Some crops cannot be mechanized

- Topography does not allow use of machines in some areas

- Lack of service centers for machinery

- Lack of spare parts

- Low return from some crops

- Inadequate capital

- Conservativeness of farmers to adopt changes

- Political instability

- Inadequate extension services

- Thick vegetation may hinder mechanization

PRE-REQUISTE TO MECHANISATION

Educate farmers on the benefits of mechanization

- Improve on land tenure system

- Effort to develop simpler and less expensive machines

- Develop simpler and more adaptable machines to do different jobs on the farm

- Mechanization should start with crops economically  viable

- Encourage farmers to own farms collectively

- Government should open agricultural engineering workshops in all towns [DFI]

- L / subsidies should be given to farmers to purchase suitable agricultural implements

- Improve on marketing and market of the products

- Train enough local  artisans to operate and maintain the machines

- Encourage local fabrications of machines and their spares

SOURCES OF FARM POWER

- Power is the rate of  doing word, If machines are to be  use to do work it must have a source of power, In the farm there need for power for

- 1; tractile work ie work done through grip on the ground and develop motion of pulling and pushing

- 2;stationary work ie work done when the machine is at a fixed place

SIX POSSIBLE SOURCES OF FARM POWER

HUMAN POWER

This is human labour use to accomplice various form of task on the farm

MERITS

- It’s cheaper than other sources of power

- Provide employments

- Suitable for small scale production

- Its suitable where there is high topography

- Does not require    specific skills

- Its readily available

- Can work without supervision

DEMERITS

- Human efficiency decline with increase in work

- Not suitable for large scale production

- Its affected by health condition of the worker

- It’s expensive in the long run because its slow

ANIMAL POWER

Man has trained animals to help supply power on the farm. Eg of animal trained includes; cattle, donkeys buffalos etc

MERITS

- No skill is required

- Can transport heavy load than man power

- Operate well when land is fragmented

- Operate on large acreage of land than man power

- Initial cost to purchase and maintain is lower than engine

Demerits

- Require land for grazing

- Less powerful than engine

- Health of animal affect the power

- Some customs discourage animal work

- Animal have shorter life span

FACTORS DETERMINING THE AMOUNT OF POWER PRODUCE BY ANIMAL

- Breed of animal

- Training level of animal

- Feeding level of animal

- Age of the animal

- Type of York used. round collar York is better than straight wooden York

CONDITION NECESSARY FOR THE SUCCES OF ANIMAL POWER

- Availability of animals

- Relatively flat land

- Light soil

- Short vegetation

- Absence of parasite / diseases

- Suitable breed of animal

- Availability of pasture

ENGINE POWER

Power is supplied by motorized machines eg tractors, mowers, hedge trimmer etc

MERITS

- It saves time

- Less laborious

- More efficient

- More varied products can produce

- Has longer life span

- Dose not suffer from fatigue

DEMERITS

- Require skill

- High initial cost  / maintaining

- They may create unemployment

ELECTRIC POWER

Sources of electricity may be from hydro electric power or thermal electric power.

MERITS

- Easy to construct

- They are lighter

- Easy and ready to start

- Operate quietly

- Produce uniform and smooth power

DEMERIT

- Initial  cost of purchasing and installation of equipments is expensive

- Its operation require skill labour

- There is risk of electrical shock

WIND POWER

Wind provide power   to do light jobs by use of wind mill

Wind power can be use for grinding, winnowing, etc .Its performances depends largely on the size of the wheels and the strength of the wind

MERITS

- Requires little capital investments

- Its easily available

- Its  free

DEMERITS

- Its unreliable

- High cost of purchasing wind mill and its accessory

- Man has no control over wind

SOLAR POWER

It’s got from the sun light; it’s a vital element for photosynthesis in green plants.

Heat from the sun is use for drying most crops prior to processing and storage

Solar energy can now be stored by use of photo voltaic cell [module] that produces electricity direct from the sun

MERITS

- Solar energy abundant esp. .in the tropics

- Its supply is free

- It require less lab our

- Require less skill

DEMERITS

- Equipments for storage is expensive

- Its effectiveness depends on weather

- Man has no control over the sun

FARM ENGINE

An engine is a machine or a device which convert the heat energy of some combust able  mechanical materials into usable power

Fuel   normally has stored energy which can be utilized

So heat engine convert chemical potential energy in fuel into mechanical energy which causes the wheel for eg to run

HEAT ENGINE

[1]    EXTERNAL COMBUSTION ENGINE

In this type of engine fuel is burnt outside the cylinder. In order to convert the chemical potential energy into mechanical energy eg Steam engine where fuel is burnt anywhere as long as it turns water into steam which will be directed into the cylinder

[2]      INTERNAL COMBUSTION ENGINE

In this type of engine the conversion of potential chemical energy into mechanical energy takes places in the cylinder and the power is supplied to the piston

MERITS OF INTERNAL COMBUSTION ENGINE

- More efficient because high percentage of fuel is converted into usable energy

- The engine is lighter than external combustion type

- Its smaller than external combustion engine

- It has less time to spent on preliminary, while for external there is need for loading, lighting , etc before the engine starts

- Operation needs less attention

- It can be made into various sizes

COMPONENTS OF AN INTERNAL COMBUSTION ENGINE

[A]     CYLINDER/ ENGINE BLOCK

This is the basic supporting portion of the Engine power units

It is made of cast iron because it’s cheap and easy to manufacture

[B]     CYLINDER

Is an air tight chamber with one end close by the cylinder head and the other end by the piston.

It’s also called the combustion chamber. Its inside the cylinder where fuel is burnt to convert the chemical potential energy in fuel into mechanical energy. It may be one or more in an engine

[C]     CYLINDER HEAD

Is a removable part which seals the top of the cylinder block.  On it is found valves system.   It’s made of high grade cast iron in order to withstand high temperature within the cylinder as a result of combustion

[D]     GASKET

It’s usually filled between the cylinder head and the cylinder block

Its function is to maintain air tight seal between the cylinder block and the cylinder head.

Its either made of copper or asbestos, because asbestos can withstand high temperature and copper is a good conductor of heat

[E]      PISTON

It moves up and down the cylinder ie makes a reciprocating motion

It’s necessary in order to maintain good compression, thus preventing leakage of fuel. It’s also conduct away heat from the inside the cylinder as a result of fuel combustion. It’s usually made of alluminion to make it light

[F]      CONNNECTING ROD

- It connect the piston to the crankshaft

- Help in transmission of power during power stroke to the crankshaft

-It has tow ends ie small end and the big end

- Its connected to the piston by the geagdion pin

[G]       CRANKSHAFT

-It’s a twisted metal rod which provide power from engine to the belt pulley and wheels

- Its twisted ness converts the reciprocating motion of the piston into a rotary motion, thus

Providing a means of doing various types of work

-Its fitted extra weight to cause a balance against vibration

-Its enclose in an air tight case called crank case

[H]       FLY WHEEL

-Its fitted to one end of the crankshaft to assist in turning

- It maintain the speed of the engine at the interval power is not produce because of inertia

Force

-The size of the fly wheel depends on the number of the cylinder. The more the cylinder the

Smaller is the fly wheel and vise-verse

[I]      CAM SHAFT

Is fitted with cam lobs which help in opening and closer of the valves

[J]      CRANK CASE/ OIL SUMP

-                     - Support and enclose the lower part of the cylinder block

-provide reservoir for lubricating oil

-It severs as a mounting point for accessories eg starter mortar, oil pump, ignition coil etc

[K]      IN-LET VALVES

It allows the entry of fuel air mixture in case of petrol engine or clean air in case of diesel

Engine during induction stroke

[ L ]       EXHAUST  VALVE

Allows the exits of exhaust gases produce during combustion of fuel air mixture in the

Cylinder

[M]          VALVE SPRING

It’s found on the valve and help in closing of the valve

[N]          PISTON RINGS

[a]compression ring

Its place in the upper groove at the top of the piston

FUNCTION

- Retain compression

- Reduces friction between the piston and the cylinder wall

-      Transmits away heat from the piston

[b]   OIL RING

Its place in the lower groove of the piston

FUNCTION

- Have slots [holes] for the return of excess oil during cylinder wall lubrication

- Transmit heat away from the piston

[O]        SPARK PLUG

Produce an a electrical spark that jump across its terminal to ignite compressed fuel

During the power stroke in the petrol engine

[P]        PUSH ROD [ROCKER ARM]

It’s along rod driven by the camshaft to open and close the valves

NB    [ i]      Ring clearance is the space left between the piston rings and the cylinder wall

[ii]     Piston clearance is the space left between the piston and the cylinder wall to enable the

Piston move up and down. It should not be too big because it will lead to loss of

-       Compression

- It will lead to loss of lubrication oil

- Lead to piston slap

EVENTS THAT TAKE PLACE IN AN INTERNAL COMBUSTION ENGINE

There are four events [strokes] viz

- INDUCTION STROKE

This is the time fuel air mixture is taken inside the combustion chamber in case of petrol   engine and clean air in case of diesel engine

- COMPRESSION STROKE

This is when fuel air mixture in case of petrol engine is compressed while in diesel engine

Its only air which is compressed

- POWER STROKE

Is when fuel air mixture is compressed to maximum and it explode [ignite] by the spark

Plug. While in diesel engine, it’s when air is compressed to maximum and a spray of diesel is

Made on it through the injector nozzle and it explode to produce power

- EXHUAST STROKE

It’s the events which help to eliminate burnt gases from the combustion chamber

TWO STROKE CYCLE ENGINE

This is the type of engine that complete one cycle in only two stroke of the piston

It has on valves but instead there are openings or ports located in the cylinder wall which gets covered and uncovered as the piston moves up and down the cylinder.

The events are group into two, viz induction/ compression and power/ exhaust stroke

OPERATION OF TWO STROKE CYCLE ENGINE

- INDUCTION STROKE / COMPRESSION STROKE

At the beginning of the cycle the piston is at bottom dead center [BDC] as it moves up wards, it uncover the inlet port and the fuel air mixture is drawn into the carburetor

Into the crank case.

As the piston continues up wards it compresses the fuel air mixture in the combustion

Chamber

When the piston reaches top dead center [TDC] a spark will be produce across the

Terminal of the spark plug that ignites the compressed fuel air mixture

2      POWER /EXHAUST STROKE

From TDC the piston moves down wards to BDC as it moves down wards it uncover the

Exhaust port and burnt gases is eliminated out. As the piston continuous down ward it uncover

The transfer port and fuel air mixture from the crank case is transferred into the cylinder

Through the transfer port and it deflected upward by the deflector piston to avoid fresh fuel air

Mixture mixing with burnt gases

MERITS OF TWO STROKE ENGINE

- Cheap and easy to maintain

- Economical in fuel consumption

- Good for small scale jobs

- Can be use in a wide range of farm land

DEMERITSOF TWO STROKE ENGINE

- Produce less power therefore weak for heavy work

- Not good in fuel and oil utilization

- Are small in size because they are mainly air cooled

- Slow at work

FOUR STROKE CYCLE ENGINE

They engines which produces power after every four stroke of the piston

NB  ; There are two types of four stroke, viz

- SPARK IGNITION TYPE

-It uses petrol as fuel

-fuel is mixed with air in the carburetor and supplied to the cylinder during induction

- Spark plug is use to ignite the compressed fuel air mixture

2     COMPRESSION IGNITION TYPE

-They are diesel engine

-during induction only air is taken into the cylinder

-after full compression a spray of diesel is made on heated air through the injector

NB; It does not use spark

OPERATION OF SPARK IGNITION ENGINE

[A]   INDUCTION STROKE

The piston will be at TDC and starts to move down ward to BDC. The inlet valve opens and the

Exhaust valves remains closed and fuel air mixture is drawn into the cylinder

[B]    COMPRESSION STROKE

From BDC the piston moves up ward to TDC. Both valves remain closed. The upward

Movement of the piston compresses the fuel air mixture

[C]      POWER STROKE

When the fuel air mixture is fully compressed a spark will jump a cross the terminal of the

Spark plug and ignite the fuel air mixture, thus producing power. Power produced will force

The piston to move down wards transmitting power to the connecting rod and to the crank

Shaft, making it to begin rotating. Both valves remained close

[D       EXHAUST STROKE

Exhaust valve opens while inlet remains closed. Piston will move up wards from BDC to TDC

Eliminating burnt gases

OPERATION OF COMPRESSION IGNITION ENGINE

[A]       INDUCTION STROKE

Inlet valve opens while exhaust valve remain close. Piston moves down ward from TDC to

BDC and clean air is drawn into the cylinder from the air cleaner

NB; only air is taken in.

[B]      COMPRESSION STROKE

Both valves remain closed. Piston moves from BDC to TDC and compresses air in the

Cylinder to maximum

[C]      POWER STROKE

Both valves remains closed after full compression. The injector pump will make a fine

Of diesel on the compressed air through the injector nozzle and it will explode producing

Power which exert pressure on the piston forcing it to move down ward to BDC

[D]       EXHAUST STROKE

Exhaust valve opens while inlet remains closed. Piston moves up ward to TDC thus

Expelling burnt gases out

MERITS OFFOUR STROKE ENGINE

- Produce a lot power

- Efficient in fuel/oil utilization

- Exhaust gases sufficiently expelled out

- Has more or heavier crank case to absorb vibration

- Can perform a wide range of farm operation

DEMERITS OF FOUR STROKE ENGINE

- They have high initial cost

- They are expensive to maintain

- Operation need skilled operator

- Their use is limited by hilly areas

SIMILARITIES BETWEEN TWO STROKE AND FOUR STROKE

- Events are still four though combine in two stroke

- Both can use spark ignition except in four stroke diesel engine

DIFFERENCES BETWEEN TWO STROKE AND FOUR STROKE ENGINE

TWO STROKE                                                                                     FOUR STROKE

- No valves but uses ports                        -         uses valves

- Two events combine in one                   - each event done independently

- Has no fly wheel                                       - has fly wheel

- Has transfer port/deflector piston       - does not have

- Has two stroke in one cycle                   - four stroke in one cycle

- Fuel enter via crank case                        - fuel does not enter crank case

- Inefficient expulsion of burnt gases     - efficient expulsion

- Has balance weight                                 - no balance weight

- Uses only petrol                                       -uses both petrol and diesel

FIRING ORDER OF AN ENGINE

This is the order in which power is produce in a multi cylinder engine

The cylinder of multi cylinder engine are arranged to have their power stroke succession

FIRING INTERVAL

IS the distance moved by the crank shaft in degree between two successive power stroke

NB; Each movement of the piston from one end to anther makes 180 degree

For a two stroke cycle engine the firing interval is 180x2 stroke = 360 degree ie when the engine has only one cylinder. But if it has more cylinder then the firing interval would be shorter ie 360 divided by the number of cylinder

For four stroke cycle engine the firing interval is 180x4stroke =720 degree ie if the engine has only one cylinder. But if it has more cylinders it would be 720 divided by the number of cylinder.

NB. The common firing order for a four cylind1er engine are; 1,2,4,3 and 1,3,4,2 while for six cylinder engine are; 1,3,2,6,4,5 – 1,3,5,6,4,2 – 1,4,5,6,3,2 – 1,4,2,6,3,5

The above can plotted on a panet square as illustrated below

Cylinder no     1       2         3       4                            order   1,2,4,3        1       2      3      4      cylinder no

STROKE         1   P        E        I        C                                   STROKE      1      P        E      I        E

2      I      C           P    E                                                       2     C        P     E         I

3    E          I         C    P                                                       3     E      I          C       P

4    C         P       E      I                                                        4    I         C        P      E

ORDER         1,3,4,2

ENGINE TERMINOLOGY

1       BORE                   Is the diameter of the cylinder

2       ENGINE SIZE      Is got by multiplying the Bore by stroke

3      STROKE               Is the maximum length of travel of the piston from one end to another in one

Direction

4      PISTON DISPLACEMENT (PD)      Is volume between TDC and BDC

5     CLEARANCE VOLUME (CV)          Is part of the cylinder ie volume between TDC and cylinder head. Its

Also called combustion chamber

6     TOTAL CYLINDER VOLUME (TCV)    Is sum of PD and CV

7     COMPRESSION RATIO (CR)            Is the ratio of TCV to CV

ENGINE SYSTEMS

There are six different systems that make the engine to perform its work effectively viz

- Cooling system

- Lubrication system

- Fuel system

- Air supply system

- Electrical system

- Transmission system

{A }            COOLING SYSTEM

It’s the system which helps to remove excess heat produce in the engine as it runs

CAUSES OFOVER HEATING IN ENGINE

- Lack of water in the radiator

- Non functioning thermo start

- Loose fan belt

- Faulty water pump

- Accumulation of dirt in the radiator grill

- Broken or loose horse pipe

- Low oil level in the sump

- Faulty radiator cup

- Leaking radiator

- Accumulation of dirt inside the radiator tubes

EFFECT OF OVER HEATING

- Expanding and melting of engine components

- Loss of power since some parts expands

- Valves will burn

- Gas leakage due to expansion of components

- Cracking of the cylinder head

- Per-ignition

- Dilution and contamination of lubricating oil

- Increase engine wear and tear due to rapid deterioration of engine oil

- Enlargement of valve seals and base

DIAGRAMME OF WATER COOLING SYSTEM

HOW IT WORKS

- Cool water is sucked from the bottom radiator tank by water pump

- Cool water enters the water jackets in the engine blocks and absorbs heat from the engine

- Heated water rise up and is force through the top horse to the radiator

- Water then moves from the top tank to the bottom tank through radiator tubes

- As heated water moves down through the radiator tubes heat from it will be remove by the circulating air drawn in by the fan

NB.   FAN;                      Draws in air which cool the engine

THERMO START    Regulate the temperature of the engine

DRAIN CAP             Is use to remove water from the radiator when it’s dirty

FILLER CUUP          Is use to fill in water

TEMPERATURE GAUGE    Indicates the temperature of water in the engine

MAINTAINCES OF THE COOLING SYSEM

- Check and top water level

- Any leakage in the system must be repaired

- Fan belt tension must be kept tight

- Radiator must be filled with clean and soft water

- Drain dirty water from the radiator

- Filler cup should be kept tight

- Replace won out fan belt

- Check and replace broken horse pipe

- Replace the pump if it’s not working

- Lubricate water pump weekly

- Flush the system twice a year

WHY IS WATER IMPORTANT AS A COOLANT

- Its cheap and easily available

- It has high specific heat capacity

- It has low freezing point

- It has high boiling point

- It’s a solvent, so it help to clean the engine

EFFECT OF RUNNING ENGINE WHEN IT’S TOO COOL

- Incomplete combustion leading to excess carbon deposit

- Increase cylinder wear and tear

- Poor circulation of engine oil

{ B }                 LUBRICATION SYSTEM

Its main purpose is separate the two metal surface with a film of oil in order to reduce friction and wear between the two moving surfaces

COMPONENTS OF LUBRICATION SYSTEM

- Oil sump;  It reserves lubricating oil

- Oil filter;   it purifies oil as it leaves the oil sump

- Oil ways;   are holes drilled in the cylinder head to convey oil at the required pressure

- Oil pump; it’s immersed in oil sump. It pump oil round the engine

LUBRICANTS

Lubricant is something which reduces friction between two surfaces in the machinery

Eg;    (i) oil –Is thin and use in engine, gear box, air cleaner, hydraulic system, differential unit, break system etc

(ii) Grease;   is thick and use in the Steering, chain sprocket, bearing and axle

PROPERTICES OF AGOOD LUBRICANT

- Low viscosity ie ability to flow

- High flash point ie should take long to ignite

- Ability to pour ie ability to flow at low temperature

- Good body  ie should be thick enough

- Good film strength ie ability to resist pressure

- Detergent ie ability to clean

- Oiliness ie smooth and slippery

CLASSIFICATION OF LUBRICANTS

Lubricants are classified according to the thickness and allotted number by the society of

Automotive engineers (SAE)

NB    Thin oil has low sae number and thick oil has high SAE number

Eg        SAE 40&30 are thin and use in small engines

SAE 40&90 are thick and use in the transmission system

SAE 120&140 Are very thick oil and use for bearing

ADDITIVES IN OIL

These are substances added to the oil to keep oil viscosity stable as the oil temperature changes

- Detergent ;      it plays a roll of cleaning

- Dispersant;     keep oil suspended in the oil way without sticking to the oil way piston etc

- Antioxidant ;   reduces the effect of oxidants which thicken oil

- Anti corrosive; protect the engine from corrosive action of water and other substances

- Anti wear  ;      reduces the rate of wear and tear of the moving surfaces

USES OF OIL

- Reduces friction

- Cools the engine

- Reduces wear and tear

- Removes dirt and small metal particles which usually chip off when the engine is in operation

- Prevent rusting

- Absorb noise from the engine

- Act as shock absorber

- Use as hydraulic fluid

- Act as a seal between the piston and the cylinder wall

CAUSES OF OIL CONTAMINATION

- Un burnt fuel

- Water produced when fuel is burnt and added to oi

- Dust / dirt from air cleaner

- Broken metallic particles

- Rust particles from rusty engine parts

- Carbon deposit

- Excessive heat in the engine

- Dirty oil filters

TYPES OF LUBRICATION

- SPLASH LUBRICATION SYSTEM

Oil is carried from the sump to the connecting rod and piston by a scooper attach on the connecting rod. Some oil goes into the reservoir above the bearing then it flows down by gravity

- OIL MIST LUBRICATION

Oil is mixed with fuel eg in 2 stroke engine. The oil mist is provided when the mixture of oil is burnt and used to lubricate the moving parts. 20part of petrol is mixed with 1 part of oil

- PRESSURE OR FORCE FEED LUBRICATION

-Oil driven around the engine through drill under pressure generated by an oil pump

-The oil pump is driven by the crank shaft.

Oil is driven from the sump through a strainer to the oil filter element.

- Filtered oil then passes to the main bearing, cam shaft bearing and big end bearing,

Valves, gears and other parts of the engine are lubricated as oil runs back to the

Sump.

-    Relief is fitted into the system to protect the pump from over loading esp. at high

Speed by bleeding out some pressure

-      The crank case also as a vent (breather) that prevents build up of pressure in the

Sump.

FUNCTION OF THE BREATHER

- Prevent build up of pressure in the crank case

- Prevent water vapour and exhaust gases from building up in the crank case

- Allows air to drawn into the crank case when the engine is cool

DIAGRAME OF A FORCE FEED LUBRICATION

OIL PRESSURE

Tractors have oil pressure gauge which show whether oil pressure is low or high. Oil        pressure should be about 2.8 kgs/cm3

CAUSES OF HIGH OIL PRESSURE

- Relief valve stuck in close position

- Blocked oil ways

- Oil too thick

- Blocked oil filters

CAUSES OF LOW OIL PRESSURE

- Low oil level in the sump

- Blocked oil ways

- Too thin oil

- Worn out oil pump

- Worn out engine bearings

- Weak relief valve stuck in open position

- Blocked oil filter

MAINTAINCES OF LUBRICATION SYSTEM

- Use correct type of oil

- Replace oil filters when you change oil

- Replace used oil regularly

- Do not use contaminated oil

- Drain oil when the its still hot to avoid sticking to the engine parts

- Check the oil level regularly using a dip stick and top if necessary

- Seal off any leakage in the system

- Clean the crank case according to the manufactures instruction

{ C }                  FUEL SYSTEM

Is the system that helps to supply fuel from the fuel tank up to the combustion chamber.

The diesel fuel system is different from petrol fuel system

FUEL SYSTEM OF DIESEL ENGINE

It’s made up of the following components which aid fuel to flow from the tank up to the cylinder

- Fuel tank      -    It stores fuel  and its location above allows for fuel to flow under gravity  to

The sediment bowel.

- Sediment  bowel    -  It removes large pieces of dirt from fuel

- Fuel lift pump        -  It add more pressure on the fuel so that it can flow up to the injector

- Fuel filter                 -  Makes the final cleaning of fuel as it continuous to the injector

- Injector pump        -  It meter fuel and makes it jump to each atomizer at a correct time.

It also has the bleeder point to bleed out air which could have continued

In the fuel.

- Atomizer          -    Change a jet of fuel into a fine spray and introduce it into the respective

Cylinder having full compression of air

FUEL SYSTEM OF PETROL ENGINE

It’s made of the following components

- Fuel tank                - It stores fuel  and fuel flow from it to the sediment bowel under gravity

- Sediment bowel   -  Help to settle larger particles from fuel

- Fuel lift pump      -   Help to maintain constant supply of fuel to the carburetor

- Fuel filters            -   makes the final cleaning of fuel

- Carburetor           -    It three major functions namely

- Mixes fuel and air to form fuel air mixture

- Atomises  (break) fuel air mixture into a fine spray

HOW IT WORKS

- Receives fuel through needle valve operated by float

- Float maintain the level of fuel in the carburetor

- Clean air enter the carburetor from the air cleaner control by the throttle

- The choke control amount of air entering the carburetor  while the float control fuel entering into the carburetor

ILLUSTRATION OF DIESEL FUEL SYETEM

-

ILLUSTRATION OF PETROL FUEL SYSTEM

DIFFERENCES BETWEEN PETROL AND DIESEL ENGINE

DIESEL ENGINE                                                                                   PETROL ENGINE

- Uses diesel as fuel                                                       -   uses petrol as fuel

- Fuel ignited by compression                                      -  fuel ignited by spark plug

- No carburetor                                                              - has a carburetor

- Has no spark plug                                                         - uses spark plug

- Has injectors                                                                 - no injector

- High compression ratio                                              -   low compression ratio

- Produce more smoke                                                  - less smoke

- Is more expensive                                                       - less expensive

- Low fuel consumption                                                - high fuel consumption

- Is more stronger                                                          --less stronger

- Only air taken during induction                               -  fuel air mixture taken in during induction

- Has few starting problems                                        - more starting problems

- Produce a lot of noise                                                - produce less noise

MAINTAINCES OF FUEL SYSTEM

- Check air cleaner weekly

- Use correct grade of oil in the air cleaner

- Check and clean wire mesh in the air cleaner by dipping in paraffin or petrol

- Change fuel filters regularly

- Seal off any leakage in the system and pipes tightly secured in place

- Injector pump and carburetor should be service regularly

- Clean the injector if block and replace won out once

WHY IS DIESEL ENGINE COMMONLY USE ON FARMS

- They are economical in fuel consumption

- It produces bigger power

- Diesel is cheaper than petrol

- It has few starting problems

- Can operate without a battery

- It cheaper to maintain

{D}                 AIR SUPPLY SYSTEM

This system plays a role to clean and supply air to the engine. Air is clean by air cleaner which is of two types viz; Dry type and Wet type.

FEATURE OF A GOOD AIR CLEANER

- Able to supply clean air into the engine

- Durable

- Easy to clean

diagram of a dry type air cleaner

Mechanism of operation of a dry type air cleaner

- When a vehicle is on motion air current is created

- Dry type air cleaner has a container with an end open for trapping air and the other end closed

- Inside it is dry paper with perforation to clean air

- Air current inside the container is force to swirl

- Air is then filtered through the dry paper

- Dust particle and other materials is forced to fall off

- Clean air then passed to the engine

MAINTAINCES OF DRY AIR CLEANER

- Use compressed pressure to blow out dust and other materials from the paper

- Use correct amount of pressure to avoid damage of the paper

- Don’t wash since its just paper

- Don’t apply pressure when the paper is wet

WET TYPE {OIL BATH} AIR CLEANER

DIAGRAM OF A WET TYPE AIR CLEANER

HOW IT WORKS

- Air is drawn in and passes through a pre-cleaner which remove larger particle of dirt

- Fairly clean air passes through the central pipe into the oil bath

- Some particle get trapped in the oil

- More clean air leaves via the oily wire mesh where all dirt are removed

- Clean air leaves the cleaner through the out let pipe to the engine

MAINTAINCE OF THE WET TYPE AIR CLEANER

- Check and top the oil level weekly

- Change oil when it becomes dark

- Swirl the wire mesh periodically in paraffin to remove dirt

- Clean the pre-cleaner off larger particles

- Ensure all joints are tight to prevent leakage

- Use recommended type of oil

- Clean the sediment bowel regularly

{E }              ELECTRICAL SYSTEM

It gets its power from the battery

FUNCTIONS

- Provide a spark in the petrol engine to ignite fuel air mixture

- Provide power for lighting headlamp, indicators etc

- Provide power for starting devices on the engine

- Provide power for the operation of horn

- Provide power for the operation of wipers

- Provide power for the operation of various gauges on the dash board

WHAT IS A BATTERY

It’s a device for storing electricity that is generated by chemical action. It consist of two metal plate one –ve and another +ve which are kept apart by hard rubber separator immersed in an electrolyte usually dilute

Sulphuric acid. All are in a casing made of hard plastic.

The +ve plate is made of lead dioxide while –ve plate is of sponge lead

NB.   When external circuit is connected to the batter y it generates electricity but when DC is passed through a battery it will be charging the battery.

CARE OF A BATTERY (MAINTAINCES OF ELECTRICAL SYSTEM)

- Top up electrolyte level with distil water if low

- Replace electrolyte if its specific gravity falls to less than 1.27

- Recharge if power is weaken

- Oil or grease the terminals to avoid corrosion.

- Vent should be kept clean

- Ensure the fan belt tension is tight to keep the battery charged.

- Empty electrolyte if the battery is not to be use for long and place it upside down on wood

- Clean the battery terminals

- Top of the battery should be kept clean and dry.

- Connect the battery terminals correctly

- Replace cracked wires.

- Battery should be fixed tightly to prevent damage due to vibration.

FLOW OF CURRENT FROM THE BATTERY UPTO THE SPARK PLUG

HOW IT WORK

- Ignition switch is turn and the engine is turn over by the starter mortar

- Current from the battery flows to the primary circuit and create a magnet in primary winding.

- The cam opens the contact breaker point causing the primary circuit to break and goes back to the coil

- The break in the primary circuit induces a high  voltage of 7000 -10000 volts in the secondary circuit

- The current then flows through heavily insulate wire via the rotor to a plug in the cylinder in which compression is taking place

- The high voltage create a spark across the terminal of the plug and it ignites the fuel air mixture

- The engine

- will then start to run.

NB.   DYNAMO (ALTERNATOR) OR GENERATOR

It has a coil in it which is run by the V-BELT to produce a magnetic field, ie

Electricity for recharging the battery.

When battery is fully re- charged cut out regulator will reduce power output of the dynamo.

Dynamo produces DC while Alternator produces AC.It can be maintain by ensuring good     tension of the drive belt.

IGNITION COIL, step up the voltage of power from the battery.

Failure of the coil ignition system: wrong clearance gap of the plug, burnt plug gap electrodes, low battery charge, broken wire, dirty battery terminal,  dirty spark plug, faulty ignition coil .

STARTER Motor, It turns the fly wheel using the electricity it gets from the battery

{ F }                               TRANSMISSION SYSTEM

Is the system which helps to transfer power from the engine to the rear wheels or driving shaft. It converts the reciprocating motion of the piston into the rotary motion for moving the wheels.

It has the following components: clutch, gear box, differential, wheels/tyres and axle

THE CLUTCH ASSEMBLY

It consist of some friction plates and it’s operated by the clutch plates to ease gradual connection for engagement of the gears

FUNCTION:

- Connect or disconnect the driving shaft to and from the engine

- Helps in changing gears

- Helps the operator to stop the vehicle without stopping the engine

- Helps the operator to take off gradually and smoothly

- Helps to provide power from the engine to the power take of ( PTO ) shaft

GEAR BOX

It has wheels with teeth around their surfaces. Every gear has different number of teeth

FUNCTION

- Enable the driver to select forward or reverse movement to suit the operation

- Helps to alter the speed ratio

- Enable the driver to stop the vehicle without suddenly stopping the engine

- Enable power  from the engine to be applied to do the work

DIFFERENTIAL

It’s composed of a set of pinion gear and crown gears. They are smaller gears on the output end of the gear box and help to make power move from the output gears to the wheel sin a right angle and this convert the mechanical energy into a rotary motion

FUNCTION

- Changes the direction of the drive to the right angle so that power is transmitted to the rear wheels

- Adjust the speed of the drive so that the operator can work at a slower speed than the engine speed

- Enable in the negotiation of the corners by allowing the near side wheel to turn slower than the off side wheel

WHEELS AND TYRES

It enable the vehicle to move and do work. It allows maximum possible grip (traction )

WAYS OF INCREASING GRIP IN TYRES

- By reducing pressure in the tyres

- Using larger tyres

- Ballasting ie adding water into the inner tube

- Use of metallic wheels but not on public road

- Increasing the number of rear wheels

- Using four wheel drive in the vehicle

- Use of new tyres

- Use of chain wheels

PROBLEMS OF TOO MUCH PRESSURE IN THE TYRE

- Reduction in traction

- Discomfort to passages

- Tyres can burst

FACTORS AFFECTING LIFE SPAN OF TYRES

- Level of inflation ie too low or too much

- Bad opening and fitting esp. with sharp tools

- Driving on sharp objects

- Poor protection from sun light

- Over loading

- Exposure to oil and other petroleum products

- Poor and bad breaking

- Use of weak quality tyres

CARE AND MAINTAINCES OF WHEEL AND TYRES

- Inflate to recommended weight ( pressure )

- Fit wheel cup to prevent entry of dirt

- Check tyres regularly for any damage

- Change tyres periodically

- Nuts / bolts should be checked and refastened daily

- Keep vehicle out of sun to prevent deterioration of rubber tyres

- Keep tyres away from petroleum products

AXLE

It connect the rear wheels and help to transmits power from the differential to the rear wheels

POWER TAKE OFF (PTO)

It’s connected to the main gear box. It uses its special gear to regulate the engine speed in order to make the tractor run stationary machines Eg : Threshers, water pump, grinding machine, and can also be use to operate rotary machines such as mowers, combine harvesters, sprayer, seeders etc

HYDRUALIC PUMP

It provides oil under pressure to operate the three point linkage and other hydraulic equipments.

MERITS

- Help to ease lifting of implements

- Require little force from the operator

- It’s not bulky

- Facilitate easy turning

DEMERITS

- Require continuous checking of oil using a dip stick

- If it fails then the steering and the clutch will also fail

- Require frequent change of oil

MAINTAINCES OF THE TRANSMISION SYSTEM

- Oil must be kept at the correct level in the gear box

- Change at specific interval as require by the manufacture

- Ensure all parts are lubricated

- Check oil level always

- Clean the filter regularly

TRACTOR

Tractor were introduce to farms to help in the process of mechanization

FACTORS CONSIDERED WHEN PURCHASING A TRACTOR FOR FARM WORK

- Adaptability to work on the farm

- Cost of the tractor

- Horse power of the tractor

- Size of the farm

- Availability of spare parts

- Fuel and oil consumption

- Efficiency at work

- Durability of the tractor

- PTO and belt pulley speed

- Guarantee given by the seller

- Ease to mount and to dismount implements

- Ease to use and maintain

MAINTAINCES / CARE OF THE TRACTOR

- Check the level and quality of oil

- Check fuel level

- Check level of electrolyte in the battery

- Grease the movable parts

- Check fan belt tension

- Check level of water in the radiator

- Check and clean air filters if dirty

- Check the pressure in the tyres

- Check and tighten bolts and nuts

- Open the sediment bowel and clean

- Check the breaking system

- Start the engine to check for normal functioning

- Before starting a tractor ensure that the shut off valves and tap is open

COMMON PROBLEMS IN ENGINE

( 1 )  ENGINE FAILING TO START

- Weak battery

- Loose wiring

- Starter mortar won out

(2)    ENGINE SWITCHES ON BUT DOES NOT FIRE

- No fuel in the tank

- Faulty fuel filter

- Weak battery

- Faulty spark plug

- Carburetor jets blocked

- Defective ignition coil

- Engine over chocked

- Incorrect ignition timing

(3)      ENGINE STOP SUDDENLY

- Faulty ignition coil

- No fuel supply

- Too much load on the engine

- Defective bearing on the crank shaft

(4)    ENGINE PRODUCE TOO MUCH SMOKE

- Black smoke means incomplete combustion of fuel

- Wrong type of fuel used

- Excess oil in the sump

- Won out oil rings

- Loose bearings

- Won out valves

(5)   EXCESS OIL CONSUMPTION

- Oil level too high

- Wrong grade of oil used

- Too high engine speed or prolong idling

- Loose or won out bearing

(6)   EXCESSIVE FUEL CONSUMPTION

- Prolong operation in low gears

- Carburetor wrongly set

- Excessive carbon deposit

- Dirty air  cleaner

- Over loading

(7)  ENGINE RUN ERATICALLY

- Spark plug dying

- Fuel  getting finish

- Wrong gears applied

- Over chocking

TILLAGE EQUIPMENTS

They are tools used for inverting the soil and burying the weeds

REASONS FOR TILAGE ( CULTIVATION )

- To kill weeds

- To loosen the soil for easy water infiltration

- To loosen  soil for easy aeration

- To improve root penetration

- To soften the soil for easy germination and sprouting of plant materials

- To ease other subsequent management practices

- To control [pest by burying and exposing their eggs, laver  and adult to predators

- To bury crop residue

MOULD BOARD PLOUGH

Diagram of an ox mould board plough

Functions of each part

- Beam       -  use for attachment of al other parts

- Handle   -   use for steering the plough

- Handle stays  - to keep the handle in position

- Mould board  -  help to invert the furrow slice

- Hake       -   help to regulate the depth rod

- Frog        -  provide attachment for mould board, share, and land side

- Share     -  use for cutting the furrow slice

- Land side  -    stabilizes the plough

- Draft rod   -   is where the chain is connected

- Depth rod   -     use to adjust the depth and width of the plough

- Land wheel   -     regulate the depth and reduces resistance while ploughing

- Link ( hitch )   -  is use for hitching the chain

Diagram of tractor mould board plough

MERITS OF MOULD BOARD PLOUGH

a)         Give good inversion of the furrow slice

b)         Require less skill

c)          Require less power

d)        Operate at uniform depth

e)       Produce relatively clean seed bed

f)         Can be use for inter row weeding

g)         Has better penetration into the soil

DEMERITS OF MOULD BOARD PLOUG

- Cannot work in stony and stumpy areas

- Its rigid and can easily break down

- Cannot manage dry hard sol

- Because of uniform depth penetration , it can form hard soil pan

- Its more expensive to maintain

MAINTAINCES OF MOULD BOARD PLOUGH

- Check the condition of the share ( tighten or straighten if necessary )

- Ensure the share is sharp

- ubricate all moveable parts

- Remove soil / vegetation from the plough after work

- Grease the mould board and other parts touching the ground if the plough is not on use

- Repair and replace won out parts

WHY OX CULTIVATION SUCEEDED IN EASTERN AND NORTHERN UGANDA

- Large herd of cattle

- Light soil

- Short grasses

- Relatively flat land

- Few livestock parasite and diseases

- Presents of adversary  services

LIMITATION OF OX CULTIVATION

- Cattle rustling and raiding

- Oxen can also get tired

- Animal can also fall sick

- Animal are slow at work

- Its only limited to land preparation, seeding, weeding and transportation

DISC PLOUGH

PARTS AND FUNCTION OF THE DISC PLOUGH

- CROSS SHAFT / TOP LINK  - Is use for connecting the plough to the tractor

- MAIN BEAM                       -  Is use the attachment of other parts of the plough

- DISC                                     -   Is use for cutting and inverting the furrow slice

- SCRAPER                             -    Is use for removing soil from the disc

- SHANK                                -   Is use for suspending the scraper

- FURROW WHEEL              - Is use to control the depth of the plough

MERITS OF THE DISC PLOUGH

- Can easily slide over obstructions

- Can be use in hard dry soil

- Work well in all type of soil

- Cheap to maintain

- Its quick in operation

- The blade can be sharpen when won out

- It control soil erosion by burying the weeds poorly

DEMERITS OF ADISC PLOUGH

- Require skill to operate and maintain

- It cannot bury rubbish well

- Leave the field in a very rough state

- Expensive to purchase

- It  is heavier than mould board plough

- Cannot be use for secondary cultivation

MAINTAINCES OF A DISC PLOUGH

- Tighten bolts and nuts

- Sharpen the blade by grinding or rolling

- Regular greasing

- If not on use coat the blade with oil

- Repair broken parts

- Do not leave the plough outside

MOUNTING APLOUGH ON THE TRACTOR / HITCHING APLOUGH ON ATRACTOR

- Reverse the tractor with lower link raised above the level of the cross shaft towards the plough until the hand lower link is correctly placed for attachment.

- Lower the linkage until the left hand lower link is at the correct height

- Brake the tractor and dismount ( get down of the tractor)

- Attach the left hand lower link and secure the link pin

- Using the leveling box fitted to the right hand lower link and necessary the cross shaft and adjustment screw ,align the right hand link, fit and screw it with the link pin

- Fit the top link between the head stock and the tractor adjusting as necessary.

- Remove the tractor and rise the implements using the tractor hydraulic system

- Avoid excessive swing of the  implements that may force the tractor

PLANTING IMPLEMENT

Planting refers to the placement of planting materials at a particular depth in the soil. The common planting equipment is the row crop planter

Diagram of row crop planter

ROLES OF ROW CROP PLANTER

- Open up soil for placement of seeds

- It meter seeds

- Placing seeds in the furrow

- Covering the seeds

- Firming the soil  around the seed

QUALITY OF A GOOD ROW PLANTER

- Able to plant seeds at uniform depth

- Able to place seeds at uniform interval

- Able to plant seeds of different sizes accurately

- Able to work at a reasonable speed

- Able to maintain its accuracy even when seeds are few

MERITS

- Quick and save time

- Plant seeds at uniform depth

- Cover and firm the seed uniformly

- Good spacing for easy weeding

- Seed placement is accurate

- No wastage of seed

- May also be use to apply fertilizer at the same time

- Less lobour is required

FARM STRUCTURES

These are physical constructions that are put on a farm to aid in the production process.

They includes: fences, cattle dip, farm building, cattle crush, spray race, milking parlous, water tanks etc.

CONSTRUCTION MATERIALS

TERMS USE IN RELATION TO MATERIALS

HARDNESS         - Is the ability of materials to resist cutting, abrasion and indentation

MALLEABILITY - Is the ability of material to remain extended in all direction when

Subjected to compression force

DUCTILITY   -      Is the ability of a material to be drawn into a thin wire by a tensile stress

TOUGHNESS   -    Is the ability of a material to withstand shock load with deformation

BRITLENESS    -    Is the ability of material to break other than bending

NB: When a material transmit a force it resist deformation by internal forces called stress which may be in form of,

- COMPRESSIVE  STRESS    - It a force that tend to shorten materials

- TELSILE STRESS                 -   It a force that tend to stretch material and increase it in

Length

- SHEARING STRESS            -  Is a force that tend to make adjacent part slides in relation

To one another

ABILITY OF MATERIAL TO RESIST STRESS IS CALLED STRENGTH AND THEY ARE IN THREE FORMS, VIZ

Compressive strength       -   is the ability of material to resist forces that tend

Shorten it

Tensile strength                  -  is the ability of materials to resist  forces that tend

To increase in its length.

Shearing strength               -  is ability of the material to resist forces that tend

To make it adjacent part slide in relation to one

Another

METALS AS BUILDING MATERIAL

MERITS

- Its durable

- Its strong

- Does not decay

- May be recycled

- Not eaten by termite

- Resistant to fire

- Resistant to wear and abrasion

DEMERITS

- They are expensive

- Increases risk of lightening accident

- Not easy to work on and mould

- Its heavy

- Not readily available

- May be subjected to rust

WOOD AS A BUILDING MATERIAL

SOURCES

- Natural forest

- Planted forest

- Local wood

- Individual trees on the farm

- Imported wood

- Industrial wood companies

MERITS

- Easy to obtain

- Relatively cheap

- Easy to work with

- With good treatment, its durable

- Have good appearance

- Its lighter compared to steel

- It has good insulating ability

- It’s a traditional material

DEMERITS

- It can be eaten by termite

- It can easily rot

- It can easily catch fire

- It has short life span

- It can develop cracks

WOOD TREATMENT

Wood must be treated and protected against termites and rotting before they are use by: creosote, old engine oil, dieldrin, tanex etc

METHOD USE

- PAINTING / VANISING

Paint is thin pigment that provides an opaque coloured finish. They are of two kinds    Eg oil paint and latex paints. While vanishes are transparent gum use for coating wood as a finish

- HOT AND COLD SOAKING

- The Burt of the wooden pole is immersed in a preservative in the tank

- The tank is heated for two hours to nearly boiling point

- Moisture in the wood cell will expand  and evaporate

- Wood then cool while still in the preservative

- The cell moisture contract and draws in the preservative

- SAP DISPLACMENT

- Peel off the back of freshly cut poles

- Place poles in the preservative in the dram

- Keep poles in the preservative for at list 5-6 days

- As the sap in the wood evaporates its replace by the preservative

- VACUUM / PRESSURE TREATMENT

- Wood is place in the container of preservative

- Vacuum is induced in the wood by pressure and the preservative is drawn into the wood.

SEASIONING (DRYING) TREATED POST

There two ways of seasoning timber viz: Air seasoning and solar seasoning

SOLAR SEASONING

Is when fresh and treated timber is dried directly under the sun. But it’s not a good system

AIR SEASONING

Is where treated timber are pilled and dried under the shade. Timber is pilled in such a way that it gives space in between for free air circulation. Circulating air removes evaporating moisture from the timber

PRECAUTION TAKEN WHEN DRYING TIMBER BY AIR

- Provide a roof to protect the timber from sun shine

- Pile timber on the ground above the ground

- Separate timber pile using wooden rod to allow passage of air through

- All support and stickers should be place close to withstand weight to avoid whapping

- The pile should be parallel to the ground to avoid sliding and bending of timber.

FENCES

Fences are farm structures that form a barrier to livestock and human movement or wild animals

MERITS OF FENCING              -      Control breeding of animal

- Control diseases and parasite

- Facilitate pad docking

- Add beauty to the farm

- Add value of the farm

- Reduces labour requirement on the farm

- Help in animal isolation

- Help to ensure efficiency use of land

- Mixed farming can be practice

- Prevent land dispute

- Reduces interference from intruders

- Seal off dangerous part of the farm, hence reducing accident

- Some life fence can be use as forage by livestock

- Life fence is a source of fire wood

- Some provide shade on the farm

- They may act as wind breaker

DEMERITS OF FENCEIN

- Deny animal from feeding freely

- Life fence may harbor pest

- Occupy land that could be use for crop or livestock

- May promote weed growth

- Makes mechanization difficult

- Expensive to construct

TYPES OF FENCES

- LIVE  FENCES

Is made of planted plant like: kier apple, Cyprus, sisal, tick berry, bougainvillea, euphorbia, cassia, wait a bit thorn

MERITS

- Act as wind breaker

- Easy to establish

- Beautiful when maintain

- Require less capital

- Source of fuel

DEMERITS

- Harbour pest and diseases

- Take long to establish

- Transporting planting material is laborious

- Some are dangerous Eg sisal

- Needs pruning all the time

- Not reliable in continuity

- Some are prisoner to livestock

- ELECTRIC FENCE

It’s a fence use in strip grazing, where plain live wire is use with low voltage of electricity to restrict animal movement in the pasture land.

MERITS

- Control the movement of livestock and wild animal

- Fence animal handling areas

- Increase crop production by protecting it from animals

- Optimum stocking rate can be achieved

DEMERITS

- Its costly for small scale farmers

- Less effective in dry season

- DEAD FENCES

They are fences made of non living material. Eg post and wire fence, wooden rail fence, trench fence and stone fence.

-    Stone / concrete fence are made of stone or brick joined by mortar to form a fence

-   Woven wire fence is use to confine small animals

-  Barbed wire fence is use to confine larger animals

-  Trench fence are dug trenches dug along other fences to prevent the straying animals. Its

Common around the game parks

MERITS

- If properly maintain can last longer

- Easy to construct

- Not affected by season of the year

DEMERITS:

- Need continuous care and maintainace

- It might be dangerous to livestock

COMPONENTS OF BARBED WIRE FENCE

a.        BARBED WIRE: they can be of low or high tensile, high tensile is made of thin shiny wire but very strong while low tensile is made of thick wire but can easily break.

b.      KING (STRAINER) POST: Is big and strong post always put at the corners or gate

c.       STANDARD POST: Are ordinary post use between the corner posts to hold wire

d.      DROPPER: are small post fitted between the wires to prevent it from sagging

e.      STRUTS: Are support put on the kingpost and standard post

PROCEDURE OF ERRECTING BARBED WIRE FENCE

- Clear the area

- Locate the corners

- Make a straight line using a string from corner to corner

- Fix standard post 5-6 meter a part

- Fix wire starting with the lower one

- Number of the strands depends on the number and docility of the animal

TOOLS USE IN INSTALLATION OF WIRE FENCE

- Earth auger : is use for boring the hold for the post

- Pliers (cutter) :is use for cutting or bending wire

- Wire strainer : is use for stretching the wire

- Rammer : is use to ram soil under the post firmly

- Staples (U-nail) : is use for fixing wire on the post

- Tap measure: is use for spacing the post

- Others are : an axe, cross cut saw, crow bar, pangar, slashes and wheel barrow

CALCULATION:

Mr. Obutu has a rectangular piece of land measuring 1500m x 600m around which a four      strand perimeter barbed wire fence is to be constructed given that the length of the barbed wire roll is 600mand the space between the fence posts is 5m and no gate should be provided.

Calculate,

- Number of fence post required

- Number of rolls of barb wire required

- Number of staple required

- If the cost of one roll of barb wire is 60,000= calculate the total cost of barbed wire required

Solution

Establish the perimeter of the land

P                   =          2(L +W)

=           2(1500+600)

=          3000+1200

=           4200M

The spacing between fence posts is 5m therefore

Number of post                      = perimeter divided by space of 5m +one post covering

=     4200/5 +1 =841 post required

- Number of rolls                     =  perimeter x number of strands divided by length of roll

=   4200 x 4 divided by 600m

= 28 rolls

- number of staple                   =    number of post x number of strand

=   841 x 4

=   3364 staples

- cost of the rolls required    =   number of rolls x cost of a roll

=   28 rolls x 60,000=

= 1,680,000=

ANIMAL HNDLING STRUCTURES

CATTLE CRUSH:

Is a farm structure use to restrain animal so that certain operation can be done on them.

USES OF A CRUSH.

- Use when caring out hand spraying

- Use when treating animal

- Use when caring out artificial insemination

- Use when dehorning

- Use when checking animal temperature

- Use when examining sick animal

- Use when milking stubborn anima

MERITS.

- Can be use for a varieties of operation

- Economical for small scale farmers

- No long distance movement is required

- Sick animal can also be sprayed

- Cheap to construct

- Farmers can control the strength of the accaricide

- No risk of spread of diseases

- It can be constructed using local available material

DEMERITS

- Not durable

- Very slow

- Accaricide may not cover animal body fully

- Waste accaricide

- Calves may not be handle

STRUCTURE OF A CATTLE CRUSH:

SPRAY RACE

Is a farm structure specifically designed for tick control. Animal walk though a confine (race) where pipes with nozzles at certain interval and angle are fitted. Animal are showered with a spray of accaricide through the nozzles

STRUCTURE OF A SPRAY RACE

MERITS:

- Many animal can be handle within a short time

- Less laborious

- Better coverage of animal body with accaricide

- Less risk of accident

- Easy to control the concentration of the accaricide solution

- Sick and pregnant animals can also be sprayed

- No wastage of accaricide

- Small animal can also be sprayed

- Easy to change accaricide

LIMITATION

- Its operation need power

- Operation need skilled man power

- Not economical for small scale farmers

- In wet weather nozzles tend to be blocked

CATTLE DIP

A dip is a farm structure which use to subject animal to the accaricide solution to control ticks

TYPES OF DIPS

MACHAKOS DIP

Is a shallow dip having about 3 steps only. Animals walk through it and it’s only the legs and belly of the animal that shall be in the solution. The back shall be out and somebody must be there to scoop the solution and pour on the back and other part not covered with the solution.

This type is mainly found in Kenya but not commonly in use

PLUNGH DIP

Is of a bigger capacity and deeper than a machakos dip. Its capacity varies from 130000 – 14000 liters of dip wash. Animal walk through it in single file and jump into the dip wash one after another where they are completely submerged into the dip wash and they swim across and walk out

DIAGRAM OF APUNGH DIP

PARTS OF THE DIP:

- Collecting yard : is the space where animal gather before the actual dipping

- Entrance race :  is a narrow corridor or crush that lead animals into the dip tank

- Foot bath           :  is found along the entrance race filled with clean water for cleaning the    hooves  of animals as they walk along

Dip tank             :   it contain accaricide solution

- Dip splash wall : It return the accracide that splashes when animal plunges in the solution

- Cat walk side   :  it about 0.5m wide, it help the attendance to walk and guide animals that fail to

Swim

- Dip roof           :   it keep off rain water from the solution

- Exit ramp        :   it made of concrete gentle steps which animal use for climbing out of the tank

- Exit race         :   Animal walk out through it as accaricide drip down from their body and return

Back to the tank. It has one hole open only during dipping to return the dripping

Accaricide into the tank and another which is open only when not dipping to divert

Running water away from the dip tank

PRECAUTIONS TAKEN BEFORE DIPPING ANIMALS

- Sick, pregnant, and small animals should not be dipped

- Do not dip animal when they are tired

- Give animal drinking water before dipping

- Move animal slowly into the dip to avoid stress and injury

- Do not dip animal when it’s about to rain

- Dip wash should be at a right concentration

- Do not use the dip wash for too long

- Foot bath should be fill with clean water

- During dipping open the hole leading into the dip tank

- Have a rope to rescuer animals that would fail to swim across

- Dip animals in the morning hours to avoid the effect of sun heat

- Dip animal every two weeks

- Dip all cattle on the farm

- Cattle should enter into the dip in a single file

ESSENTIAL OF AN EFFECTIVE DIP

- It should be on a well drained land

- Present of permanent water supply source

- Cement the collecting and drainage yard

- Foot bath should be filled with clean water

- The flow of entrance race / exit race should be made of concretes and slop backwards to the tank

- The dip should be in the center of the grazing land

- Dip should not be in the depression

- The flow and the wall should be leak proof

- The wall should be smooth to return the splashing accaricide

- There should be soak pit for empting the solution

- The flow of the dip should not be too slippery

- Jump off step should be gentle to enable animal gently slide into the dip tank

- Exit ramp should have gentle steps to enable animal easily come out

- Exit race should be long enough to allow animal dry before going to the pasture

- Dip tank should be big enough to contain required capacity of accaricide solution

- It should have a cat walk side

MAINTAINCES OF A DIP

- Keep dip wash at correct level

- Fill foot bath with clean water

- Close hole returning the dip wash if not dipping

- Make sure dip tank has no crack

FACTORS THAT MAY REDUCE THE EFFECTIVENSSOF DIPPING

- Failure to follow the dipping routine

- Mixing weak accaricide

- Improper mixing of accaricide

- Leaking roof

- Dipping in the rainy day

MERITS OF DIPPING

- Effective tick control

- Many animal can be handle

- Less laborious

- The solution can be use for a long time

- No wastage of the solution

- No source of power is needed

- Cheap to run communally

DEMERITS OF DIPPING

- Sick and pregnant animal cannot be dipped

- Expensive to construct

- Not economical for small scale farmers

- Can transmit livestock diseases

- Leakage of dip wash may take place

- Empting the dip tank is laborious

- Accident may occur during dipping

EXAMPLES OF ACCARICIDES USE

Gammertox, coopertox, toxaphane, supona, delnav, supamix, bacdip, decatic, pfizertox, supona-extra, Stella done, spoton etc

FARM BUILDING

It comprises of different kinds of buildings found on the farm such as: home stead, calf pen, rabbit hatch, pigsty, maize crib, milking parlous, stores etc

IMPORTANCE OF A FARM BUILDING

- Provide comfort to human and animal by acting as a place of abode

- Increases production by reducing crop and animal loss

- Increases efficiency and ease in the management on the farm

- Guard against bad weather on stored crop, machines and tools

- Reduces labour requirement on the farm

- Hel to buffer stock the products

- Reduces losses on the farm

- Increases the real estate value of the farm

- Help to improve the quality of produce through processing

FACTORS CONSIDERED WHEN SITTING FARM BUILDING

- Accessibility

- Water supply

- Elevation ie should be on a raise ground

- Should be on less productive land

- Nearness to the source of power

- Should be in the centre of the farm

- Should be near the home stead

- Land for expansion

- Fuel stores should be isolated

- It should have a beautiful canaries

- Avoid wind direction

CONSIDERATION IN DESIGINING BUILDING FOR FARM ANIMALS

- Construct a slanting flow to allow fluid to drain away quickly

- The flow should be made of concrete not too smooth nor too rough

- It should have a roof and a wall

- It should be well ventilated

- Provide bedding ie litters on the flow

- The wall should be strong and smooth

- Light should be provided

MERITS

- Protect animals from bad weather

- Security against theft, wild animals etc

- Easy management

- Reduce spread of  diseases

- Encourages close supervision

- Reduces feed wastage

CONSIDERATION WHEN PUTTING STORAGE BUILDING

- Should be in an open space to avoid vermin and fire

- Should be rodent/ termite proof

- The wall should be free from crack

- It should allow for easy ventilation

- The wall should be water proof

- Easy to clean

- Should be in a secure place

- Should be on a strong foundation

MERITS OF STORAGE BUILDING

- Serve as a buffer stock

- Allows for easy harvesting

- Reduces labour requirement

- Protect produce from bad weather

- Sorting and grading of produce is facilitated

- Enable keeping of crop with moisture content easy in the shade

- Provide security for produce against theft

BUIDING MATERIALS

MORTAR

It’s a plastic mixture of water, sand and cement

USES OF MORTAR

- Use for bonding brick

- Use for plastering the wall

- Use for flow finishing

- Use for rough casting the wall

CONCRETE

Is a mixture of sand, cement, aggregate and water

USES OF CONCRETE

- Use for making fence post

- Use for making flow and wall

- Use for making gabions and water channels to prevent soil erosion

- Use for making water and feed troughs

MERITS

- Durable

- Resistance to weather and termite

- Easy to clean

- Fire resistant

- Cheaper than steel

- Superior in strength to wood

DEMERITS

- It’s expensive

- Its bulky

- Not traditional ie needs skilled labour

- Low in tensile strength

- Not easy to repair

- Not salvageable

Other materials are: plastic, brick, thatching grass, tiles etc

FUNDATION OF A BUILDING

It’s the seat of the house it should be strong enough on the ground

PROCEEDURE OF MAKING A FOUNDATION

- Measure the site

- Dig the site to the required depth

Remove all the organic matter or loose soil

- Mark the foundation with pegs

- Use spirit level to check horizontal level

- Throw broken stone/bricks into the excavated site

- Pour concrete on top into the site

- Ram gently the site

- Lay final  layer of concrete footing

- Allow it to set

- Damp proof course (DPC) is put 15cm above the ground level to prevent water soaking the wall

DIAGRAM OF THE FOUNDATION

THE WALL STRUCTURE

This should be constructed in such a way that it supports only the vertical load since they are not able to support side loads.  Spirit level is use to check the horizontal level while plum bob is use to check the vertical levels

THE ROOF

The roof covering is commonly supported by Trusses; Tie beam runs from wall to wall, Rafters makes a triangular structure above the tie beam and supported by the strut. After the trusses (kacolia) are made and in the position of purlins are nail horizontal on the rafters and the roofing materials thatch

DIAGRAM OF THE ROOF

FARM WATER SUPPLY

USES OF WATER ON THE FARM

a)     For domestic use eg cooking, bathing, washing etc

b)     For irrigation

c)     For livestock to drink

d)     For mixing agro chemicals

e)     For mixing building materials

f)      For recreation eg swimming

g)     For cooling heat engines

h)     For cleaning animal hooves

i)       To generate power (electricity)

j)      For processing farm produce

k)     For aquaculture eg fish farming

CAUSES OF WATER SHORTAGE

- Inadequate rainfall

- Deforestation

- Drainage of water bodies

SOLUTION TO WATER SHORTAGES

- A forestation to fight desertification

- Diversification of agriculture

- Growing of short term maturing crop

- Keeping draught resistant livestock

- Construction of dams

- Establishment of food reserves

- Insure ring farming activities

- Use sunken seed bed

- Pollution control and treatment of polluted water

- Mulching of gardens to preserve soil moisture

SOURCES OF WATER USE IN THE FARM

Bore hole, rivers, lakes, swamps, dams, springs, wells, streams, rain water, and ponds.

WATER STORAGE

Water is stored in several structures eg

- House hold containers eg drums, jerry cans, tins, pots, gourds etc

- Dams like : water troughs, ponds, valley dams

- Storage tanks like: over head tank, underground tank and house tank

OVER HEAD TANK

Is usually made of galvanized steel to it from rusting and its raised above the ground to ease water flow out of the tank by gravity. It has over flow pipe to allow excess water flow out

ARTS OF THE OVER HEAD TANK

- Tank body;    is made of galvanized iron. It hold  water

- Support/ stands  :   holds the tank body up

- Inlet pipe  :    deliver water from the water source to the tank

- Outlet pipe :  supply water to other parts of the farm where water is needed

- Drainage pipe:     to allow dirt out of the tank during tank cleaning

- Over flow pipe:  to allow excess water flow out

- Safety pipe :  to release excessive pressure from the tank

- Air vent:     to release air in the water so as to avoid air overflowing out in the outlet pipe which would otherwise cause air bubbles and blockage of the pipe

WATER TREATMENT

- HEAT TREATMENT

Water is boiled at very high temperature and allowed to cool and kept in containers

FILTERING USING SAND

Water is filtered through sand and is capable of killing germs and removal of other

Foreign materials

CHEMICAL TREATMENT

Steps:

- Water is sieved at the point of intake. This remove large impurities

- Softening of water is made in the small tank where its mixed with sodium bicarbonate

- Coagulation/ sedimentation: alum is added to facilitate coagulation and sedimentation. Water stays in this tank for 36 hours to kill bilharzias. The tank is then open to remove bad smell/ colure and aeration

- Filtration: water passes into the filtration tank where all remaining solid particle are removed

- Chlorination:  filtered water enter into the chlorination tank where small amount of chlorine is added to kill germs

- Storage:  treated water is then stored in large tank before distribution for use

DIAGRAMM OF OVER HEAD WATER TANK

CARE / MAINTAINCE OF FARM TOOLS

- Proper handling of the tools during use on the farm

- Each tool should be use for the right purpose its meant for

- Clean the tools after using it

- Grease tools if they are not to be use to avoid rusting

- Keep tools away from children

- Carry out regular servicing of the tools

- Keep all tools in the store.

AGRICULTURAL ECONOMICS

Economics is the study of how man chooses to use scarce resources to satisfy his unlimited wants. Every society faces problems of scarcity of resources which may be, natural or human.

However, economics help the producer to know the followings

- What to produce

- How to produce

- Where to produce from

- For whom to produce

- How much to produce

- How and when to buy / sell

PRODUCTION

This is the process in which resources are transformed into products usable by consumers. Therefore it’s the creation of utility.

Where utility is the ability of the goods to satisfy human wants.

FORMS OF UTILITY

- Form utility: The goods must be in the nature which is able to satisfy human wants

- Place utility:  The goods must be available in the place in which its required to satisfy wants

- Time utility:    The goods must be available at the time its required to satisfy human wants

FACTORS OF PRODUCTION

Production is the transformation of input into output and it has the following factors involved:

Land, Labour, Capita, and Entrepreneurship.

(A)     LAND

Land is a gift of nature and it refers to all natural resources eg minerals, forest, water bodies etc. Land is not homogeneous, it’s immobile, its supply is fixed and its quality can be improved by use of fertilizer / reclamation. Payment for land is called “RENT”

(B)         LABOUR

Labour is human effort both physical and mental directed towards the production of goods and services.

IMPORTANCES OF LABOUR

- It utilizes land and capital for production to occur

- It determine the value of goods eg labour intensive products are very expensive

- Production ultimately aim at satisfying labour since laborious are human for whom the goods are intended

- The quality and motivation of labour force determine the output from a productive venture

- It create capital by accumulation of reward it get from offering labour

CHARACTERISTICS OF LABOUR

- Its human effort

- It cannot be separated from the owner

- Labour is mobile

- Is not inherited unlike land and capital

- Labour has high opportunity cost ie it takes a lot of time, resources and effort to train labour

- Labour has a will of its own unlike land and capital. It has a capacity to make it own decision or modify instruction given to it.

FACTORS AFFECTING LABOUR SUPPLY

- Total population and the size of the labour force in it

- Wage rate

- Health of workers

- Average hours of work

- Geographical mobility of labour ie by transport, communication and advertisement

- Level of skill require

- Working condition

- Political stability

- Nature of the job

- Retirement benefits

- Rural urban migration

- Attitude of people towards work

NB: Payment for labour is called “WAGE”

FACTORS THAT DETERMIN THE EFFICIENCY OF LABOUR

- Availability of transport

- Provision of housing

- Appropriate remuneration

- Specialization

- Adequate supervision

- Provision of meals at work place

- Giving appropriate work load

- Proper handling of employees by employer

- Equipment being used in  production

- Provision of entertainment. This reduces stress while at work

(C)       CAPITAL

Capital are resources made by man to facilitate the production of other goods/ services

FORMS OF CAPITAL

- Real capital: is a stock physical asset  eg roads, railways, building, tractors etc

- Money/ financial capital: is money invested into the business to produce more goods

TYPES OF CAPITAL

- FIXED CAPITAL: they are durable capital like: roads, railways, building, tractor etc

- Semi fixed capital: is productive in a short run  eg breeding livestock,  hoes etc

- Circulating capital: is regarded as working capital. It include cash use for regular expenditure on the farm

ROLES OF CAPITAL IN PRODUCTION PROCESS

- It increases the effectiveness of labour force by provision of machine, tools etc

- It enable the exploration and fuller utilization of the available natural resources

- It increases the productive capacity of the nation by exploiting idle resources

- It improve the quality and quantity of the national output

- Real capital can be use as security for bank loan

- Capital can be exported to other countries to generate foreign income

- Capital help the entrepreneur to mobilize land and labour

NB: Payment for personal capital is called profit while payment for borrowed capital is called interest.

ENTERPRENEURSHIP

An entrepreneur is a manager and his work is to mobiles resources like land, labour and capital in order to produce goods and services

ROLES OF AN ENTERRENEUR

- Combine other factors of production to make production possible

- Bears risk and uncertainties

- Control management

- Make production decisions

- Carries out innovation

- Supervises production

- Raises finances and other resources through saving and borrowing

- Buys input

- Co-ordinate and plan for the enterprise activities

- He or she is the final decision maker

PRODUCTION FUNCTION

This is the physical relationship between input and output. It shows how the quantity of a particular product various with the amount of input used in a specific time period

TYPES:

- CONSTANT RETURN

Is where the amount of products (output) increases by the same amount for each additional units of input used

Illustration

Output                                          TP                              Eg        input                              output

- 100

0                                    140

20                                     180

30                                     220

40                                     260

- 300

input

2)    INCREASING RETURN

Is when each additional unit of input used result into a larger increase of output than the previous one

Illustration

output                                     TP CURVE

Eg      input                       output

- 10

- 15

- 23

- 37

- 54

3)      DIMINISHING (DECREASING) RETURN

Is situations where by when you keep on adding the input to a fixed factor output will begin increasing at a decreasing rate

Illustration

Output                                                                                      Eg          input                      output

5                             20

- 29

- 36

- 40

- 42

TYPES OF INPUT

- Fixed input : are inputs that do not vary with the level of production eg land, water sources, permanent labour etc

- Variable input: are input that keep on varying with the level of production eg farm tools ,labour seeds fertilizer etc

TYPES OF PRODUCTs

- Total physical product (TPP)  Is the total physical product produce by a given level of input

- Average physical product(APP) IS got by dividing TPP by a given level of input used

- Marginal physical product(MPP) IS extra output produced by extra unit of input

COST OF PRODUCTION

- Opportunity cost: is the cost of the alternative foregone in the production of a product

- Fixed cost(FC) is the cost that do not vary with the level of production

- Variable cost(VC) are cost that vary with the level of production eg feeds, casual labour etc

- Total cost(TC) Is the summation of both FC+VC

- Explicit cost: is the easily recognize and quantify  eg cost of feeds ,seed ,fertilizer hired labour

- Implicit cost: are cost not easily recognize and rarely valued in accounting eg personal labour

- Marginal cost: is additional cost met in the production of an extra unit of output

LAW OF DIMINISIHING RETURN

The law states that when successive unit of variable input are added to a fixed factor held at a constant output will increase at a greater rate and reaches a stage when the increase will reduce

Eg    a farmer has one acre of land and employed different quantity of labour to grow maize.

Use the data below to draw a illustrative graph for the law of diminishing return

| land(acres) | Number of men | Tpp | App | Mpp |
| --- | --- | --- | --- | --- |
| 1 | 1 | 08 | 08 | - |
| 1 | 2 | 20 | 10 | 12 |
| 1 | 3 | 36 | 12 | 16 |
| 1 | 4 | 48 | 12 | 12 |
| 1 | 5 | 55 | 11 | 07 |
| 1 | 6 | 60 | 10 | 05 |
| 1 | 7 | 60 | 8.6 | 00 |

Illustration

TPP CURVE

output

stage 1              stage 2                    stage 3     APP CURVE

irrational zone            rational zone               irrational zone

Zone (stage) 1:                                                                                                                         MPP              Input

IS irrational zone of production , resources are still under utilize so any addition of variable factor yields a greater return eg if a farmer was not using fertilizer and he start using it the yield will increase greatly

Zone (stage) 2:

This is rational zone of production where a farmer has already use the resources to maximum and any addition of the variable factor result into little increase in total product

Zone (stage) 3:

This is irrational zone of production. Its where a farmer has over utilized the resources, so any addition of variable factor result into the decrease in the output

PRODUCTION POSIBILITY FRONTIER

It shows the combination of commodities that can be provided by a society using its limited resource.

It bases its assumption on;

- Only two commodities can be produce given the level of resources eg commodities W and X

- Full employment of resources

- The level of technology is constant

Illustration                         A

Good X        E                                    B

F                                                C

O                               H          I       D

Good W

Curve ABCD is the production possibility frontier curve. All points on the curve depict a situation of full employment of resources. Any point inside the curve indicates that resources are not fully employed.  To increase output of X from OF to OE there should be a reduction in the output of W from OI to OH

NB: The production possibility frontier curve can only shift outwards if:

- Technology is improved

- Stock of resources has increased

- Better organisation of production process is put in place.

PRODUCT RELATIONSHIP

JOINT PRODUCTS:

Are products produce through a single production process EG wool/mutton, cotton lint/cotton seed, milk/butter, beef/hides, egg/chicken meat etc

COMPETITIVE PRODUCTS

Products are competitive if the increase in production of one necessitates a decrease in the production of the other given fixed amount of input. Most products are competitive in nature eg; keeping cattle and pigs in the same farm, dairy and beef production on the same farm etc.

SUPPLEMENTARY PRODUCTS

Two products are supplementary if the production of one can be increase without affecting the output of the other eg keeping chicken in a dairy farm, bean in a coconut field

Illustration             A                                         B

C                A – B Is supplementary range

B – C is competitive range

C – D is supplementary range

D

COMPLEMENTARY PRODUCTS

Production of Y1 can be increase without affecting production of Y2 in range A-B While output of Y2 can be increase without affecting output Y1 in a range D-C

While at range B-C any increase of either Y1or Y2 will affect the other.

These are products where the transfer of resources from one product to another result to an increase in both products eg growing of legume crops and legume plants, waste grain and pig enterprise.

Illustration

In the complementary range below X increases from A to H while W simultaneously increases from O to C. After point B, W will continuous to increase but X will start to decrease

Product X              H                            B                                        A – B is supplementary range

A

O                            C             Product W

SPECIALIZATION / DIVERSIFICATION

SPECIALIZATION

Is where resources are concentrated in the production of one commodity

MERITS

- Lead to mastery of the job

- Increases the quality and quantity of produce

- Lead to trade in order to dispose surplus

- Reduce wastage of resource

- Facilitate mechanization since machines for the task can be develop

- Less mental strain as a result of mastery of the job

- Increased productivity leads to reduced cost per unit of output hence reducing the price consumer have to pay

- Ease marketing  because of one or few product

DEMERITS

- Over dependence on others to provide what you have not produce

- Effect of price fluctuation

- May lead to over production of a commodity

- It leads to monotonous (boredom)

- It limits the range of commodities available locally

- Limits occupational mobility

- Leave other resources un utilize

- Farmers suffer from lack of constant income since agricultural products are seasonal

DIVERSIFICATION

Is the production of several products at the same time

MERITS

- Less risk of total loss in case of failure in another product

- Lead to wide range of product resulting to more dependant and self sustaining

- Reduces seasonality of income

- Reduces seasonal un employment

- It widen export base of the country

- Enable stable economic growth

- Enable long term planning of the economy due to stable income

- Lead to integration of farm by- product eg crop residues can be use to feed livestock

DEMERITS

- Difficult to manage combination of enterprises

- Require multi skilled labour

- May lead to spread of pest, parasite and diseases

- Difficult to market several products

RISK AND UNCERTAINTY

RISK

These are measurable variables which with a specific degree of probability can predict a situation in future.

Risk can therefore be estimated and insured against to avoid total loss.

KINDS OF RISK

- Diseases, pest, parasite out break on the farm

- Weather changes

- Theft

- Insecurity

- Fire hazard

- Land tenure policy eg short notice to the to the tenants by the land lord

- Health of the workers

- Accident on the workers

- Un availability of market

UNCERTAINITY

Are outcome that cannot be establish statistically base on the probability of their occurrences in future. Uncertainty cannot be insured ie its concern with the future nobody has perfect knowledge about.

KINDS OF UNCERTAINITY

- Technological changes

- Price fluctuation

- Change in demand and supply

- Unavailability of input

- Change in government policy

- Breach of contract

- Unavailability of labour

MEASURES TO OVER COME RISK AND UNCERTAINITY

- Insuring your properties against risk

- Having a flexible enterprise

- Diversification

- Liquidity ie maintaining a balance of cash

- Use high yielding / resistant crop and animals

- Select the most certain enterprise

- Use buffer stocking

- Improve quality of produce

- Under take saving

- Have the ability to borrow

- Apply contract farming

- Input rationing so that they are not used up at ago.

- Adapting modern method of farming

- Employ experience personnel’s

- Encourage cooperative in order to pull resources together to help the farmers

- Maintaining adequate security on the farm eg having fences, lights, etc

- Giving farmers price stabilization fund by the government

FARM RECORD

These are written information got or that transferred in the farm eg expenditure, profit etc

Importance of farm record

- Help in planning and decision making

- Enable them to know if he or she is making profit or loss

- Help the farmer to obtain loan

- Improve upon management efficiency by telling the farmer where he might have gone wrong

- Enable the farmer to remember his debts so that he can pay

- Enable the farmers taxes to be assessed correctly

- Help in profit/losses sharing in the cooperatives

- Shows the history of the farm and its development

- Provide data for extension workers

- Help in animal identification

- Provide information for selection program

- Increases efficiency in animal and crop improvement

- Help in settling estate in case of death

- Help the farmer in making insurance claim.

- Etc

Types of records kept on the farm

- Crop record

- Labour record

- Production record

- Calving record

- Health record

- Breeding record

- Financial record

- Farm historical record

- Inventory

- Feeding

MARKET DEMAND / SUPPLY

DEMAND (dd)

Demand is the amount of a given commodity a consumer is willing to buy at a given price and time.    The desire backed by the ability to pay for it is referred to as effective demand while the reverse is non effective demand

LAW OF DEMAND

It states that other factors remaining constant as price decreases the demand increases and

Vise verse.  Or it states that the quantity of a given commodity various inversely with price

DEMAND CURVE

Is a smooth curve showing the quantity of a given commodity demanded with the corresponding prices

Illustration:                 price

qtty

DEMAND SCHUDLE

Is a table showing the quantity of the commodity demanded at various prices

Illustration

Price of sugar                   quantity demanded

2000=                                        5kgs

2500=                                       4kgs

3000=                                      3kgs

FACTORS AFFECTING DEMAND OF COMMODITIES

- Price of the commodity

- Price of other goods eg substitute, complementary goods etc

- Income of the consumers

- Taste and preference

- Size of the population

- Government policy eg taxation

- Seasonal factors eg festive seasons

- Availability of transport

- Future speculation

- Nature of the goods eg necessity or luxury

- Structure of the population eg age, sexes etc

- Fashion eg people buy more of the goods that are fashionable at the time

- Level of advertisement

ELASTICITY OF DEMAND

This is the degree of responsiveness of demand to change in price.

Forms of elasticity of demand

- Price elasticity of demand: Is the percentage change in quantity demanded resulting from change in price

- Point elasticity of demand: It measures the ratio of a very small change in quantity demanded to a very small change in price

- Arc elasticity of demand: It measures the responsiveness of quantity demanded to a relatively large change in price.

Types of elasticity of demand

- ELASTIC DEMAND

Is when a small change in price causes a larger change in the quantity demanded and the co-efficient is greater than 1. This leads to increase in total revenue. The dd curve is a flatter curve slopping from left to right.

Illustration        price

- INELASTIC DEMAND

Is where a fairly big change in the price causes less than proportionate change in the quantity demanded and the co-efficient is less than 1. This leads to the fall in the total revenue. The dd curve is a steeper curve slopping from left to right

Illustration            price

quantity

(c)      PERFECTLY ELASTIC

Is a situation where any change in price will cause the demand to fall to zero and the co-efficient is infinity. The demand curve is straight and parallel to the X- axis

Illustration

price

d           d

quantity

(d)          PERFECTLY INELASTC:

Is a situation where by any change in the price of the commodity will not affect quantity demanded and the co-efficient is equal to zero.  The demand curve is straight line parallel to the Y-axis.

Illustration          price                        d

d

qtty

(e) UNITARY ELASTICITY

Is where percentage change in price is equal to percentage change in quantity demanded and the co-efficient is equal to 1

HOW TO CALCULATE ELASTICITY OF DEMAND

ED                                     =      Change in quantity demanded ÷ change in price

Percentage change in dd =       (change in demand ÷ original demand) ×100%

%tage change in price       =       (change in price ÷ original price) × 100%

FACTORS AFFECTING PRICE ELASTICITY OF DEMAND

- Availability of substitutes eg where there is close substitutes the ED is elastic and vise verse

- Degree of necessity of the commodity eg all essential commodities have inelastic demand while luxury goods have elastic demand

- Number of user of the commodity ie the more the number the more its inelastic and vise verse

- Joint demand commodities that are demanded jointly. It is the price of the major commodity that determine the elasticity of the second commodity eg cars and fuel

- Durability of the product, durable product are more elastic in demand while perishable once have inelastic demand

- Addictive and habit. addictive and habit commodities like cigarette, drugs and alcohol tend to have inelastic demand

- The level of income of the consumers, consumers with high income their demand for commodities is inelastic

CHANGE IN QUANTITY DEMANDED

This the movement along the demand curve cause by changes in the price of the commodities. Eg moving from A to C on the diagram below

Illustration

price             P3            A

P2                  B

C

P1

Q1   Q2     Q3          qtty

CHANGE IN DEMAND (DEMAND SHIFT)

This occurs when the quantity demanded at any particular price changes causing the entire shift of the demand curve either to the left or right.

This change is cause by other factors other than price         Illustration

Price

D1         D              D2

P1

D1           D                D2

Q1            Q            Q2         qtty

SUPPLY

This the amount of the commodity producers are willing and able to offer for sale at a given price and time.

LAW OF SUPPLY

It states that the quantity of a given commodity supplied various inversely with price ie

If he price is low, and other factor remain constant, less of that commodity will be supplied and the reverse is true

SUPPLY SCHEDULE

Is the table showing the quantity of a given commodity supplied with the corresponding prices

SUPPLY CURVE

Is a smooth curve showing the quantity of the commodity supplied with the corresponding prices

Illustration

PRICE                         S

S

QTTY

FACTORS AFFECTING SUPPLY OF COMMODITIES

- Price of the commodity

- Price of other commodities eg substitutes/ complementary

- Number of producer in the market

- Level of technology

- Government policy

- Season of the year

- Political stability/instability

- Gestation period of the commodity

- Cost / availability of factor of production

- Natural hazard

- Future speculation

- Transport and communication

- Goal/aim of the producer ie charity and profit motive. If the motive is charity, the suppler of the goods is not affected by the changes in price and vise verses

Change in quantity supplied

This the movement along a given supply curves resulting from the change in price of the commodity while other factors remain constant

Illustration                     P3

P2

P1

Q1    Q2     Q3

Change in supply (supply shift)

It occur when the entire supply curve shift to either left or right due to changes in other factor other than price of the commodity in question.

Illustration        PRICE                                    S1         S                   S2

P1

S1              S                  S2

Q1           Q                 Q2

PRICE ELASTICITY OF SUPPLY

This is the degree of responsiveness of supply to change in price

ES            =        CHANGE IN SUPPLY ÷ CHANGE IN PRICE

TYPES

(a)ELASTIC SUPPLY

Is when a small change in the price of the commodity causes more than proportionate change in the quantity supplied. And the co-efficient is greater than 1; the slope of the curve will be gentle.

Illustration                 PRICE

S

(b)INELASTIC SUPPLY

Is when a big change in price causes less than proportionate change in the quantity supplied. The co-efficient is less than 1 and the slop of the curve is steep

Illustration                PRICE

S

(c)UNITARY SUPPLY

It occurs when a change in the price causes an equal change in the quantity supplied.

(d)PERFECTLY ELASTIC SUPPLY

Is when price is the same at all level of output supplied

Illustration        PRICE

P1     S                       S

QTTY

(e)PERFECTLY INELASTIC SUPPLY

Is when the quantity is the same at all price levels

Illustration          PRICE                S

S

CALCULATION OF PRICE ELASTICITY OF SUPPLY

  - ES                                               =        change in supply

Change in price

- %tage change in supply        =   change in supply   x    100

Original supply

(C )  %tage change in price          =   change in price      x    100

Original price

RELATIONSHIP BETWEEN DEMAND, SUPPLY AND PRICE IN A FREE MARKET

This help to explain how price is determine in a free market by the forces of demand and supply while other factors remain constant.

Eg      price of milk (shs)    amount of milk supplied       amount of milk demanded                excess/deficit

100                                                 250lit                                            95lit                                               +155lit

80                                                    220lit                                           140lit                                             +80lit

60                                                    185lit                                            185lit                                              0lit

40                                                    120lit                                            200lit                                             -80lit

20                                                    85lit                                               240lit                                             -155lit

10                                                    20lit                                               255lit                                             -235lit

EXPLAINATIONS

A.        DOWNWARD PRESSUER ON PRICE

Here the supply of milk is plentiful but at very high price of100shs and this price affected the demand. So the consumers are only able to buy 95liters leaving excess milk 155liters. This then forced the supplier to reduce the price

B.      UPWARD PRESSURE ON PRICE

This comes as result of discouragement of the suppliers to supply because of low price causing a deficit in supply but because of low price the demand is too high leading to excess demand. This situation forces the supplier to raise the price

C.        EQUILIBRIUMPRICE

This the price at which demand equal to supply Eg price 60 where supply is 185 and demand is also 185.  It also called equilibrium quantity or point.

Down ward pressure on price

Illustration                    price                 d                        s

EP                                              E. point

s              Upward pressure on price

EQ                         qtty

IMPORTANCE OF PRICE IN AGRICULTURAL PRODUCTION

- It stimulate production

- Determine how to produce

- Determine distribution of income and wealth

- Determine distribution of product ie who consumes what

- Act as incentive to growth ie high prices encourages growth

- Facilitate full utilization of resources

FACTORS AFFECTING PRICE OF A COMMODITY

- Government policy

- Cost of input

- Marketing cost

- Amount of goods supplied

- Objective of the producer

- Level of specialization

- Quality of the commodity

- Demand of the commodity

- Price of other goods

- Market structure eg monopoly , competition etc

TRADING ACCUONT

PROFIT AND LOSS ACCOUNT

It’s a projection of sale/receipt against purchases/expenditure. It measures the performance of the business.

FEATURE OF A GOOD PROFIT AND LOSS ACCOUNT

- Heading or title starting the duration of the account

- Purchase/expenditure are entered on the left while sales/receipt are entered on the right

- Value of item on the farm at the beginning of the year(opening valuation)is entered on the purchases/expenditure side. While the value of assets at the end of the year (closing valuation) is entered on the sales/receipt side

- Net profit is when sale/receipt side is greater than purchases/expenditure side, and the difference is called net profit and it entered on the purchases side. While NET LOSS is got when purchases/expenditure side is greater than sale/receipt side. This difference is called net loss which is entered on the sales/receipt side.

Eg    a)     prepare a profit and loss account for Soroti s.s farm as at 31 /12 /2010 using the following data

Cattle bought                                                          210000

Sale of milk                                                               350000

Sale of cabbages                                                      112000

Bean in store                                                            100000

Eggs in store                                                             171000

Debts recoverable                                                   192000

Seed                                                                              50000

Opening valuation                                                   650000

Poultry feeds                                                              97000

Closing valuation                                                     750000

Drugs                                                                            96000

Debts payable                                                          100000

Rent                                                                             10000

Wages                                                                       197000

Cattle feed                                                                75000

Fertilizer                                                                     25000

PROFIT AND LOSS A/C FOR SOROTI S.S FARM AS AT 31/12/2010

PURCHASES/EXPENDITURE                                                        SALE/RECEIPT

Opening valuation     650000                           sale of milk                            350000

Cattle bought             210000                            sale of cabbages                   112000

Fertilizer                        25000                            bean in store                         100000

Cattle feed                   75000                             eggs in store                         171000

Wages                         197000                            debts receivable                   192000

Rent                               10000                            closing valuation                 750000

Debts payable            100000

Drug                               96000                                                                         1675000

Seed                               87000

Poultry feed                 50000

1500000

Net profit                    175000

Total                  =      1675000                                                Total        =       1675000

b) Prepare a profit and loss a/c for beta farm for the year ending 2009 using the following data

Sale of cattle                                                                            400000

Sale of milk                                                                              1000000

Drugs                                                                                          125000

Sale of bean                                                                              400000

Purchase of cattle                                                                    700000

Wages                                                                                       1500000

Sale of cabbages                                                                      700000

Debts payable                                                                           400000

Fuel                                                                                             396000

Seed                                                                                            30000

Opening valuation                                                                2000000

Debts receivable                                                                     300000

Rent                                                                                             90000

Purchases of poultry                                                              600000

Closing valuation                                                                   2420000

Purchase of feed                                                                     405000

Sale of bull                                                                               800000

USES OF PROFIT AND LOSS ACCOUNT

- It identifies fund generated from normal operation of the business

- It analyses the performance  of thee business

- Its an a/c in which revenue for an accounting period is appropriately matched

LIMITATION

- It does not give the full record of the transaction affecting the business

- It list only revenue/expenditure and ignore capital transaction

- It does not describe the flow of fund in and out

BALANCE SHEET

It’s a statement drawn up to show the financial position of the farm on a particular date.

OR    It’s the statement of assets and liabilities held at a particular date.

ASSET:

They are what the farm own or they are properties of the farm.

CLASSIFICATION OF ASSET:

- CURRENT ASSET: are asset held temporally in the business Eg cash, market livestock etc

- WORKING ASSET: are normally used up during the life of the business Eg land, machinery , building etc

- FIXED ASSET: are assets which have permanent or semi permanent position on the farm. Eg land, permanent buildings etc

LIABILITY

They what the business owes to others. OR ARE Claims against the farm.

CLASSIFICATION OF LIABILITIES

- Long term liability: are liabilities payable in a longer period of time eg mortgage

- Intermediate liability: are liability payable from after one year

- Current liability: are liability payables within one year.

FEATURE OF A GOOD BALANCE SHEET

- TITLE : the title must indicate the name and the date at which the balance sheet is drawn

- Liabilities are entered to the left while assets to the right

- Both liability and asset must be entered in the same order eg order of permanency or liquidity

- Net capital is got when asset side is greater than liability side and its entered on the liability side while net loss is when liability side is bigger than asset side and its entered on the asset side.

Eg   (1)     prepare a balance sheet for alpha farm as at 31/12/2011 using the following dates

Cash at bank                                                 8000

Bank overdraft                                            15000

Debts payable                                             15000

Debts receivable                                           2000

Value of land                                               40000

Value of sheep                                              5000

Value of cattle                                             20000

Value of coffee                                            45000

Long term loan                                            50000

Value of building                                         45000

Value of machinery                                    35000

Balance sheet

For alpha farm

As at 31/12/2011

LIABIILITIES                                                                                          ASSETS

Bank overdraft                          15000                                  cash in bank               8000

Debts payable                           15000                                  debts receivable       2000

Long term loan                          50000                                  value of land           40000

Value of sheep          5000

80000                                    Value of cattle         20000

Value of coffee        45000

Value of building    45000

NET CAPITAL =   120000                                 Value of machinery    35000

TOTAL          =     200000                                          TOTAL                    200000

- Prepare a balance sheet for scudeto farm as at 30/11/2012 using the following data

Debts payable                                          320000

Value of implement                          1250000

Value of crop                                        300000

Value of livestock                                300000

Bank overdraft                                     1450000

Cash in bank                                          288000

Debts receivable                                  462000

Depreciation                                           60000

Value of building                               2000000

Prepaid expenses                                600000

LIMITATION OF A BALANCE SHEET

- It’s a static image of the financial of the financial position of the business at a given date yet business keeps on changing.

- It’s the historical statement which does not show the present value of the asset

- It does not describe the flow of fund in and out of the business during a given period

- It ignores the inflationary and deflationary trend of the business.

- It ignore un quantifiable and intangible factors

MERITS OF A BALANCE SHEET

- Enable a farmer to acquire loan

- Help in decision making

- Help in assessment of taxes

- Useful in sharing profit

- Shows the farmers procession

- Shows farmers debts so as to pay

- Help in economic analysis of the farm

- Help in planning

FARM BUDGETING

It’s a detail quantitative statement of a farm plan regarding estimate and financial returns.

THE BUDGET SHOULD SPELL OUT THE FOLLOWINGS

- What to produce

- How much to produce

- How much input will be required

- Expected overall cost

- Expected return

- Expected profit and losses

HOW TO MAKE A BUDGET

- Identify your needs

- Identify your resources

- Cost each of your need

- Arrange your needs in order of priority

- Find the alternative means of satisfying your needs

- Choose the best alternatives

- A lot the resources to the priorities

- Write out as draft

FUNCTION OFA BUDGET TO A FARMER

- It help to forecast profit or losses

- Its use in decision making

- Help the farmer to control production

- Help in making effective changes

- Help the farmer to estimate required production resources in terms of labour, capital

- Help to reduce uncertainties in farming

- Encourages the farmer to efficient with the hope to meet the projected targets

TYPES OF THE BUDGET

- PARTIAL BUDGET :  Is use when planning for a smaller change on the farm

- COMPLETE BUDGET: It’s the budget that covers complete reorganization of the farm eg changing crop farm to livestock farm. It will involve costing all the fixed and variable cost as well as returns expected.

LIMITATION OF A BUDGET

1)      Lack of interest in optimizing return

2)      Lack of technical information/failure to recognize un used resources

3)    Lack of inspiration, the farmer may disregard certain potential alternative because he cannot recognize all possibilities

4)     Failure to recognize supplementary and complementary enterprises that would require less added cost by farmers

POPULATION ECONOMICS

Population is the number of people living in a particular area in a given period of time.

COMPOSITION OF POPULATION

Population is always composed of different sexes, age, occupational distribution, literacy, marriage, religions etc

AGE DISTRIBUTION

Population is always made of young, old and working class (labour force)

The distribution defers in less developed countries (LDC) from developed countries (DC) as illustrated on the age pyramid below.

- This age pyramid is for LDCS and

Characterized by many young,                                                         - 60

Moderate working class and a few old

-  40

-  20years

- This age pyramid is for DC and its                                                 - 60 years

Characterized by many old, moderate

Working class and a few young.                                                     – 40years

-   20

- This age pyramid is affected by migration

Of the working class from one region                                        -  60 years

To another in search of better

Economic projects.                                                                         -  40years

-  20years

POPULATION GROWTH

This is the measure of percentage increase in population which is always brought about by increase in birth rate and decrease in death rate. It might also be caused by increase d rate of migration.

THEORIES EXPLANING POPULATION GROWTH

(A)        DEMOGRAPHIC TRANSITION THEORY

It explains population growth from historical perspective. According to this theory population in developing countries has gone through four stages namely

STAGE 1:

Here, death rate was very high due to poor living condition ie poor medical facilities, poor sanitation, poor diet and poor technology. Birth rate is also too high due to low level of education and high demand for children as a source of family labour.

STAGE 2:

Here, birth rate is still high and the death rate is declining due to improve technology and improve standard of living (LDCS’S ) are still under this stage.

STAGE 3:

Here, death rate starts to decline due to education and improved standard of living. There is also less emphasis for family labour due to change in technology, rising status of women and greater urbanization which require small family

STAGE 4:

Here, birth rate and death rate are at minimal, almost balance fertility rate falls below zero point that is necessary for replacement. Total population has a lot of old people.

Illustration of demographic transition

BR

DR

1              2                3                    4

Time

(B)  MULTHUSIAN POPULATION THEORY

Rev. Thomas Malthus was a British economist who lived in the 19th century. HE noted with concerned that the population of Britain was increasing more rapidly than ever before and he related this to the Law of diminishing returns since the land was relatively fixed in supply. He believed it would be impossible to increase food production to match with the population growth .According to him mans Biological ability to reproduce exceeds his physical capacity to increase food supply.

ASSUMPTIONS OF THE THEORY:

- He assumes that population growth depend on food supply. When food supply increases population also will increase.

- He assumes that population grows at geometric rate ie by simply doubling while food supply grows at arithmetic rate ie by constant increase because of this he believed that at one time population would outstrip food supply resulting into famine, congestion, misery, and death thus a population trap.

NB:      He identify the following checks on population:-

- Positive checks eg wars, famine, and natural death

- Negative checks eg late marriage, family panning etc

Illustration

Population growth                                                                                                 population growth

A                                                                 B              F                            food growth

C

D                                                                                   Population trap

Food production

O                                                                                                              Time

ABC represent food supply growth.

DBF represent population growth when there are no positive checks.

DBC represents population growth when there are positive checks.

Population cannot go towards B-F because beyond B people will start dying.

CRITICISMS OF MALTHUSIAN POPULATION THEORY

- He assumes that resources like land are fixed and cannot increase in size, but he ignored the fact that land can be improved eg by irrigation, drainage, addition of fertilizer etc.

- He did not see the possibility of increasing food production through technological advancement. But today we have modern machines and improved crops / animals

- He ignored migration of people which would help to reduce population pressure on an area.

- He ignored opening up new areas for agriculture, he was only concern with Britain which having a close economy.

- He ignored the roles of trade and foreign relation in the supply of food and other resources eg WFP, Red cross etc

- He thought food is the only determinant of population growth and he ignored migration, culture, religion etc

- He assumes that any increase of income above subsistence level would lead to increase in population yet it’s not true because population always decline with increase in population due to improved standard of living

- His theory is based on agriculture as abases of economy. However developed countries have undergone transformation and industry/ commerce are systematically replacing traditional agriculture

- Malthus looked at population only in a negative perspective yet population growth can lead to development eg china, India

- He did not indicate the time when population trap would be reached.

RELEVANCEOF THE MALTHUSIAN THEORY TO DEVELOPING COUNTRIES

- Population growth in some developing countries have been checked by positive checks eg famine, epidemics, wars etc

- Malthus advocated for family planning which is now largely practices

- In LDCS scientific and technological advances have not yet penetrated so traditional agriculture is still relevant and the productivity is declining so the law of diminishing return is at work.

- The theory of optimum population is rooted in the relationship between food and the population.

UNDER / OPTIMUM/ OVER POPULATION

- Optimum population is when the available labour is adequate to combine with the available resources to give maximum output without putting too much strain on the resources.

- Under population is when labour available is not adequate to combine with the available resources to give maximum output.

- Overpopulation is where there are too many people for the available resources

INDICATORS OF OVER POPULATION

- Low per capita income

- Low standard of living

- Low per capita output as a result low ratio of asset to labour

- Shortage of land and capital

- High level of unemployment and underemployment

- Pressure on existing social services eg schools, hospitals etc

- Shortage of housing facilities

CAUSES OF HIGH POPULATION GROWTH RATE IN DEVELOPING COUNTRIES

- Cultural factors, children are seen as a  source of prestige, wealth and insurance in old age

- Improve medical care

- Polygamy

- Demand for cheap family labour

- High infant mortality rate makes people to produce many children with hope that some will die.

- High fertility rate of women in developing  countries

- Early marriages due to inadequate education and unemployment

- Low status of women many of them are poor, illiterate. Such house wives believed their role is only to produce children

- Low level of urbanization in the rural setting there is a lot of free food, accommodation, and land so the problem of having large family is not a problem

- Idleness in rural areas and to some extend in urban setting makes people look at sex as the only form of recreation.

EFFECT OF INCREASING POPULATION

+VE EFFECT

- Creation of wealth, large population is good to utilize the resources

- Increases the size of market

- Increases the size of labour force

- Leads to development of modern infrastructures in the area

- It stimulate hard work in order to cope up with the growing population

- Lead to international recognition eg china is a super power because of it population

-VE EFFECT

- Many social services will be run down

- Heavy dependence burden

- Increase expenditure on social services

- Low level of saving since much will be consumed

- High rate of illiteracy

- Environmental pollution and degradation

- Productivity of the land decreases due to population pressure on land

- Wide spread of poverty since resources will not be enough

- Leads to inflation because  there will be few commodities for very many people

- There will be congestion, diseases, famine, high rate of crime etc

POPULATION CONTROL

- Through family planning

- By giving incentive to small families and dis-incentive to large families

- Through improved education since it tend to delay marriage

- Coercive policies, force is applied through various legislative measures

- Legalizing abortion to enable women produce at will

- Encourage international migration

- Encourage celibacy this reduces the number of breeding human being

- Marriage age legislation with the aim to delay marriage

- Encourage urbanization which require one to have small population because of the cost of living

LIMITATION TO POPULATION CONTROL

- Religious opposition

- Low level of economic development

- Abortion is opposed by religious leaders

- Low level of education

EDUCATION AS A FACTOR OF DEVELOPMENT

Education is the acquisition of knowledge and skill. It can be formal or informal

BENEFITS OF EDUCATION

- Reduces unemployment which may be because of lack of skill

- It a means of social transformation

- Helps to identify talents in people

- Its an instrument of rural development

- It offers employment in education sector eg teachers, to those who make scholastic materials

- Improves the quality and efficiency of labour force

- It improves the entrepreneurial skill of people

- It produces personnel that under take research and experimental projects

- It reduces income inequality

- It check the population growth since it delays marriage

- It shape leadership quality and develops a sense of nationalism. This can help to forge national unity.

- It increases demand for scholastic materials

- Help the nation to save forex that could have otherwise be spent on hiring expatriate from other countries

AGRICULTURAL MARKETING

A market is a situation where there are suppliers with goods willing to supply and willing buyers

Ready to buy the goods at the price acceptable to the suppliers

Marketing refers to all the processes involves in transformation of raw materials into finish products which can be sold to consumers

MARKETING FUNCTIONS:

- Buying and assembling

- Selling

- Transportation

- Storage

- Processing

- Grading

- Standardization

- Packaging

- Financing all activities

- Risk bearing

- Carry out market research

- Advertising

ADVANTAGES OF PACKAGING

- Reduces bulkiness

- Reduces shrinkage and spoilage

- Help in easy handling

- Facilitate  quality identification

- Help in advertisement

- Encourage self serves thus reducing cost of marketing

- Reduces adulteration and substitution

ADVANTAGES OF PROCESSING

- Prolong the useful life of a commodity

- Reduces wastage or spoilage

- Extend the period of availability of the product and so reduces fluctuation in supply

- It reduces fluctuation in price due to constant supply

- Help to destroy toxin in the products

- Add value to the products

CHARACTERISTICS OF AGRICULTURAL PRODUCTS THAT MAKE THEM DIFFICULT TO MARKET

- Bulkiness of the products

- Perish ability  of the products

- Seasonality of the product

- Inelastic demand

- Large  number of small and independent producer

- Variation in the quality of agricultural products

- Long gestation period of agricultural products

- Competition from synthetic substitutes

- Lack of market information

- Inadequate transport facilities

- Lack of processing facilities

- Political instability

HOW TO IMPROVE THE MARKETING OF AGRICULTURAL PRODUCTS

- Construction of proper storage facilities

- Establishment of processing industries

- Establishment of cooperatives

- Improve on market research

- Improve on extension services to farmers

- Provision of credits to farmers

- Market diversification ie open new market

- Trade contract and protocols between the producers and the consumers

- Timely provision of credits to farmers

- Improvement on transport

- Diversification of farm production

- Introduction of price stabilization fund

- Use of quota system that involves restriction of production or sale

PRICE FLUCTUATION OF AGRICULTURAL PRODUCTS

This is the sudden rise and fall in the price of agricultural products

CAUSES:

- Divergence between the planned output and the actual output obtained

- Long gestation period in the production of agricultural products

- Inelastic demand of agricultural products

- Perish ability of agricultural products

- General lack alternative use of resources use in agriculture eg land, farm tools etc

- Seasonal nature of the production

- Large number of small scale farmers so they can not influence the market in their favour

- Bulkiness makes transporting it from one market to another difficult

- Weak commodity agreement in producing countries

- Variable quality of agricultural products

- Agric products forms only a small part of manufactured goods eg rubber and cars

METHODS OF REDUCING PRICE FLUCTUATION OF AGRICULTURAL PRODUCTS

- Price legislation ie setting maximum price legislation

- Establishing processing industries

- Improvement on transport infrastructure

- Introduction of buffer stocking/ price stabilization fund

- Diversification of agriculture

- Formation of international commodity body to monitor and regulate supply of products in the market and to increase bargaining power of the producers

- Market diversification ie open more markets

- Introduction of industrialization and scientific innovation to reduce over reliance on agriculture

- Technological and scientific innovation within agricultural sector to reduce long gestation period of agricultural products

- Improvement on market research

- Improvement on the quality of the products

- Formation of marketing organisations

EFFECTS OF PRICE FLUCTUATION

- Leads to unstable income to farmers

- Discourages saving

- It affect government revenue

- Causes high risk in production

- Affect the countries planning

- Makes the government unpopular due to blames from farmers

- Results into rural urban migration

- Leads to unemployment

- It forces the government to subsidize agriculture sector

- Discourage production of commodities whose price has fallen

EFFICIENCY IN FARMINY

This measures of physical and financial performance of a farm. It helps the farmer to     identify his or her weakness / strength so as to make improvement.

EFFICIENCY STANDARD

It’s a measure use to compare the performance of  two or more farms.

WAYS OF ASSESSING EFFICIENCY OF THE FARM “OR”TYPES OF FARM EFFICIENCY

- TECHNICAL EFFICIENCY:     It measures physical output per unit of input eg one farmer produce 2500kgs of maize and another produce 3500kgs , both use the same amount of land, seed, fertilizer, labour and same type of soil and same growing condition. NB This means that the farmer who produces 3500kgs has more technical skills than the one who produce 2500kgs.

- ECONOMIC EFFICIENCY:      Here the cost of production is weigh against the returns obtained. Profitability is therefore use as comparison. Eg If two farmers obtained the same quantity of simsim and sold at the same price but their production cost differ, the one who has lower production cost will have enjoyed economic efficiency.

- EFFICIENCY STANDARD:       Here a set of standard is given Eg Friesian cow is expected to give 4000- 5000 litter of milk per lactation. Now with the above set standard a farmer can compare his yield and see where his weakness is and correct.

- OVERALL EFFICIENCY:         Here efficiency is got by summing up all the profits from each enterprises then divide by the amount of capital used then get the percentage

Ie       overall efficiency         =            total profit       x        100

Capital used

- PARTIAL EFFICIENCY STANDARD:   Here a section of the farm enterprise is considered Eg

- YIELD INDEX:             It measures percentage yield against expected yield

Yield index        =     actual yield        x      100

Expected yield

- SYSTEM INDEX:         Here we compare the yield of a particular product from two similar farms Eg the yield of eggs from layers from farm A&B

Ie     system index     =   yield on farm A        X   100

Yield on farm B

NB     If the index is lower than 100% it means the farmer has to improve on his efficiency.

WAYS OF IMPROVING EFFICIENCY ON THE FARM

- Follow proper crop production and managements

- Follow proper animal production and management practices

- Mechanized production which involve use of machines

- Careful planning

- Carry out constant research to improve on crop/ animal quality and quantity

- Make prediction of the outcome

- Make a round decision at the right time

- Record keeping

- Improve on housing and sanitation

- Improve government policies that can boost farm production

- Adapting new technique and method

- Having well organised farm lay out

- Selecting proper enterprises that are less risky

- Diversification to spread risk

- Organising marketing activities to create high prices

FACTORS AFFECTING EFFICIENY IN FARMING

- Price fluctuation of both input and output managerial ability to make sound decision

- Size of the farm large farms gives better results than small farms

- Record kept good record improves on farm efficiency

- Climatic condition, changes in climate affect efficiency in farming

- Pest/ parasite and diseases affect the  yield

- Natural hazard like floods, earthquake etc

- Human factors eg skills, strength, culture etc

- Poor government polices eg taxes, credit provision etc

- Insecurity in the area

- Soil characteristics

AGRICULTURAL CREDITS

This is borrowed capital used by farmers to finance their activities. It may be in cash or in

Kind.

TYPES:

- Short term credit: -      It’s repayable within one year. It can be used to purchase simple things like; fertilizer, feeds, seed, fuel, sprayers etc. It has very high interest rate.

- Medium term credit: -   It’s repayable within 2 – 15 years. It’s normally used for minor land improvement like fencing, purchase of machinery, purchase of livestock, soil conservation, irrigation installation etc. Its interest rate is lower than short term credit

- Long term credit: -     It’s repayable within 15 – 30 years. It’s normally used for major improvements on the land like construction of permanent buildings, irrigation for perennial crops. Its interest rate is lowest than the above two.

SOURCES OF AGRICULTURAL CREDITS

- Cooperative societies

- Commercial banks

- Insurance companies

- Individual money lenders

- Traders

- Hire purchase companies

- Government scheme eg Entandikwa

- Licensed money lenders

- Micro finance institutions eg FINCA

- Informal credit from friends

- Self financing

IMPORTANCE OF CREDIT TO FARMERS

- Enable farmers to finance their activities throughout the year/ production season

- Provide capital for building up the enterprise

- Enable farmers to cope up with the seasonal pattern of production

- Help to overcome risk and uncertainties

- Help farmers to purchase long term assets eg land, machinery etc

LOAN (CREDIT)

This are borrowed capital and it’s of two types viz; hard loan and soft loan

- HARD LOAN: -       Its granted after sound security is guaranteed, its interest rate is high, no grace period is given, and large amount of money is given, Payable on a longer period

- SOFT LOAN: -        grace period given , low interest rate, no security required and its payable within a short time

REASONS WHY FARMERS FAIL TO REPAY CREDITS

- High interest rate

- Fall in the price of produce

- Misuse of credit

- Defaulting /dodging about

- Insecurity / stolen

- Poor management of the enterprise

- Failure of the enterprise due to natural hazard

- Poor timing of credit

- Ill – health of the farmer

- Poor marketability

- Short repayment period

- Death of the farmer

- Fall in demand

- High taxation

- Inflation which increases the cost of production

- Lack of trust worthiness

MEASURES TO MAKE AGRICULTURAL CREDIT MORE EFFECTIVE

- Credit lesson should be included in the extension service programme

- Commercial attitude of farmers should be develop

- Farmers should be trained on credit management

- Credit to inexperience farmers should be granted in kind

- Sound security should be demanded before credit is granted

- Farmers should be supervised

- Loan be given in time

- Provision of input to farmers at low price

- Credit should be given in portion within the production period

- Grace period should be given to farmers

- Create more market for farmers produce

- Recovery formula should be explain to the farmer

FACTORS THAT MAY LIMIT THE AVAILABILITY OF CREDIT TO THE AGRICULTURAL SECTOR

- Most farmers lack collateral security

- Most farmers have inadequate knowledge about the operation of credit institutions

- Urban location of most credit institutions

- Difficulty in supervising long term loans

- Credit from traders and friends may be limited due lack of credit worthiness

- Agricultural sector has a lot of risk and uncertainties

- Corruption and embezzlement

- Inflation in less develop countries

- Majority of people are poor resulting to low lending capacity

SUBSIDY SCHEME

A subsidy is an AID to sustain or supplement business

IMPORTANCE OF SUBSIDY TO FARMERS

- Help to stabilize the price of agricultural products

- Encourage people to invest

- Influence the pattern of  investments towards subsidized products

- It stimulate the improvement of infrastructure like roads, stores, marketing facilities

LAND TENURE

This means possession of legal right to use of land.

TYPES

- PRIVATE OWNERSHIP

The individual owner of the land gets a title for it and it become private ownership

MERITS

-it’s an incentive to improve the land

-it settles / avoid demarcation dispute

-allows for land consolidation

-safe guard the position of the local community if land is in short supply

-it can be use to get loan

DEMERITS

-  encourage sale of land

-   encourage hoarding of land

-   Land lord may get the loan and fail to pay and end up losing the land

-   It encourages unfair distribution of land

-   Land may remain idle if it’s owned by lazy person

b.    COMMOUNAL (CUSTOMARY) OWNERSHIP

Here land does not belong to an individual but to clearly defined section of the

Community eg: Clan, Tribe or nation.

MERITS

-  Every member of the community has access to land

-  No hoarding of land

-  No land fragmentation

-   No land dispute since every member knows the regulation of the use of land

DEMERITS

-  Poor utilization of the land eg overgrazing

-  Poor maintenance of the land

-  Individual farmer has no right to use the land for his own plan.

-  It difficult to acquire individual title deed for the land hence he cannot get loan

-  Difficult to control pest, parasite and diseases

-  limit the use of modern method of production.

CO-OPERATIVE LAND TENURE

It’s owned by a group of farmers who have come up together to form co-operative society.

MERITS

-  Land dispute is minimized since no individual can claim ownership

-   encourage large scale production

-   They can a title deed which they can use for getting bank loan.

DEMERITS

-    No individual incentive to work hard

LEASE HOLD LAND TENURE

Is where the state or land lord give out land to and individual to use for a specified period of time. The lease holder pays land rent to the government or land lord

The lease period is always 49years, 99 and 999years.

MERITS

-   Encourage investment in long term projects

-    no land dispute

-    those unable to purchase land can rend (lease)

-    gives income to government

-   lease holder can get loan using his title

DEMERITS

-  Lease holder may lose if the state or land lord refuses to renew the lease

-  the state can terminate the lease and compensate the leaseholder, but this will

Disorganize the farmer.

PROCEDURE OF ACQUIRING LAND IN UGANDA

- By becoming part of the collective farming group

- By borrowing from others

- By being settled or resettled by the government

- By hiring from those who has

- Through inheritance

- Through leasing from land lords or government

- Allocation by parents or relatives

- Through buying from others

- Squatting illegally on the piece of land.

SETTLEMENT AND RESETTLEMENT

This is land reform programme which involves movement of people to a new area with

The aim of boosting Agricultural production in area.

Settlement is settling people in the place which was not occupied. While resettlement is

Where people are moved from highly populated area and settled in the place which is less

Populated.

OBJECTIVES/ REASONS

- To reduce population pressure on land

- To promote agricultural mechanization

- To facilitate effective delivery of social services

- To prevent re-infestation by tsetse-flies

- To assess the feasibility and economic returns from setting up a large irrigation scheme

- To resettle the displaced persons

- To encourage self employment

- To allow a variety of agricultural activities from the various settlements.

LAND FRAGMENTATION

Is where a farmer has several small pieces of land instead of one large piece of land.

CAUSES

- Shifting cultivation

- Traditional agricultural system of inheritance

- Increasing population

- Lack of money to buy chunk of land

- Communal land tenure system

- Accumulation of land by the farmer through buying and inheritance

DEMERITS

- Waste time to move from one plot to another

- Hinders mechanization

- Farm fencing cannot take place

- Makes farm planning difficult

- Encourage communal grazing

- Difficult to control pest, parasite and diseases

- Agric extension services is difficult since farmers are scattered

LAND CONSOLIDATION

This is the pooling together of small pieces of land

MERITS

- Saves farmers time

- Makes mechanization possible

- Makes farm planning easy

- Increases efficiency in farming

- Reduces theft of farm produce

- Makes storage of farm produce easy

- Makes provision of extension services easy

- Easy control of pest, parasite and diseases/ weed.

FARMING ORGANISATIONS

EXAMPLES:

- Co-operatives

- Settlement scheme

- District farm institutes

- Agric research station

- Out growers

- Young farmers association

BENEFITES OF FARMING ORGANISATION

- Farmers share overall cost

- They are able to buy input in bulk at a low price

- Organisation eliminate middle men

- Organisation can integrate marketing processes like  package, transport etc

- Organisation can improve on management by employing skilled personnel’s

- They have high bargaining power

- Members can get marketing information

- Easy for them to export product and get better price

- Exposes members to commercial life

- Leadership/ unity develop among members

- Help to transform moral life

- Help to supply input to farmers

- Members can obtain credit facilities

PROBLEMS FACED BUY FARMING ORGANISATIONS IN UGANDA

- Mismanagement by leaders

- Government interference

- Poor service delivery

- Greed and selfishness of some members

- Lack of incentives  offered to farmers

- Competition from industrializing the economy

- Poor organizational structure

- Political insecurity in the country

- etc

GROSS MARGIN

This the difference between total revenue and total cost met in the production

Ie                   TR - TC        =   Gross margin