743/2

BUILDING CONSTRUCTION

(Project)

PAPER 2

July/August 2018

ASSHU – RWENZORI REGION ACADEMIC BOARD MOCK 2018

6 hours

ASSHU - RWENZORI REGION ACADEMIC BOARD (ARRAB)

Uganda Certificate of Education

MOCK EXAMINATIONS

BUILDING CONSTRUCTION

(Project)

Paper 2

6 hours

INSTRUCTIONS TO CANDIDATES:

This paper consists of three assignments

Answer assignment 1 and either assignment 2 or 3.

Turn Over

© ASSHU – Rwenzori Region Academic Board Mock 2018

- Figure 1 shows an outline of floor plan of a DOMESTIC HOUSE, not draw to scale.

SPECIFICATIONS:

| FOUNDATIONS: | 690 mm by 150 mm concrete strip, 1000mm deep from ground level. |
| --- | --- |
| WALLS: | All walls are 225 mm thick and 3000 mm high from floor level to the underside of wall plate. |
| FLOOR: | Cement sand screed 25 mm thick laid on 100 mm thick site concrete with a blinding layer 25 m laid on 150mm thick hard core. |
| DOORS: | D1 1200mm wide by 2100mm high glazed metallic casement double shutters. |
|  | D2 2400mm wide by 2100mm high metallic double shutters. |
|  | D3 900mm wide by 2100mm high framed, ledged braced and battened doors. |
|  | D4 800mm by 2100mm framed, ledged, braced and battened doors. |
| WINDOWS: | W1 steel glazed casement, 1200mm wide by 1200mm high with concrete window sills. |
|  | W2 steel grazed casement, 600mm x 600mm. |
| ROOF: | Double pitched at 30o and gabled, with closed eave 60mm projection and covered with iron sheets. |
|  | Purlins 75mm x 50mm |
|  | Rafters 100mm x 50mm |
|  | Tie beam 150mm x 50mm |
|  | Struts /ties 100mm x 50mm |
|  | Wall plate 100mm x 75mm |
|  | Fascia board 225mm x 25mm. |

Assignment 1: (60 MARKS)

Using the specifications and drawing shown in figure 1, draw to a scale of 1:100, the:

- Plan of the house using related symbols to show all the doors, windows, sanitary fittings, ward robes in their appropriate positions.           (20 marks)

- Front elevation in the direction of arrow H.                                         (09 marks)

- Rear elevation in the direction of arrow K.                                          (09 marks)

- Vertical section through cutting plane X –X and show all the constructional details from foundation to ridge.                                                               (22 marks)

ASSIGNMENT 2: (40 MARKS)

- To a scale of 1:10, draw the plan of course 1 and 2 of the wall in circled P in Flemish bond (4 bricks long)                                                                            (10 marks)

- Draw to a scale 1:10 in isometric projection elevation of five courses in English bond at a junction marked M (4 bricks long both sides)                          (13 marks)

- Assume the house in figure 1 was erected on loose ground. To a scale of 1:10, draw in isometric view a suitable limbering to trenches used in this soil conditions. Name the members.                                                                       (17 marks)

ASSIGNMENT 3: (40 MARKS)

- Draw to 1:10 a three centered ARCH of rise 400mm at D1 in figure 1 and label the following;                                                                                                            (20 marks)

- Striking points

- Intrados

- Key brick

- Poussoirs

- Extrades

- Springe

- Skew back

on one side show the construction details.

- Draw to a scale 1:10 a two paneled window, 1000mm wide by 1200mm high.

(10 marks)

- (i) Sketch a cross section of an open eave. Name the members.               (05 marks)

(ii) Use appropriate sketch to illustrate tiled flooring.                                 (05 marks)

END