P720/2

TECHNICAL DRAWING

BUILDING DRAWING

Paper 2

Jan-Feb 2013

3 hours

HOLIDAY WORK

TECHNICAL DRAWING

BUILDING DRAWING

Paper 2

SENIOR SIX

3 Hours

INSTRUCTIONS

This paper consists of six questions.

Answer all questions.

Where dimensions are not given suitable dimensions are to be chosen.

Accuracy and good draughtsman ship are essential.

Figure 1 shows the ground floor plan of a RESIDENTIAL HOUSE not drawn to scale. Parts A and B are roofed with a concrete slab.

# SPECIFICATIONS:

| ROOF: | Hipped roof covered with angle ridge clay tiles pitched at 45o on battens 50x50 on roofing felt and wire mesh on Rafters, purlins, & struts 125mm x 50mm onTie beams 150mm x 50mm. Trusses placed 2230mm center to center. Fascia boards/ verge boards are 200mm x 25mm. |
| --- | --- |
| WALLS: | External and internal are one and half thick brick walls in Flemish bond. |
| DOORS: | D1- 2100mm x 1500mm. casement double leaf.D2- 2100mmx900mm flush door , D3-2100x1500 panel door, D4- 2100x2400 steel double leaf door |
| WINDOWS: | W1- 1500mm x 1200mm casement metallic. W2- 1800mm x 1200mm casement metallic W3-2400mmx600mm Pompeii grills. W4- 600mmx750mm casement metallic. W5- 750mmx750mm serving hatch |
| FLOOR: | Concrete floor with 20mm cement/ sand screed laid over 100mm thick over-site floor concrete and 150mm thick hardcore. |
| FOUNDATION: | Concrete strip foundation 675mm wide by 230mm thick at a depth of 1300mm below the ground level. |

- Draw a free hand pictorial sketch of the building with corner S at the fore ground. (Use of instruments will be penalized).

2. Draw to a scale of 1:75, the plan of the house and show using appropriate symbols :

- Doors in their best positions,

- Windows in their best positions,

- The serving hatch in the kitchen.

- To a scale of 1:75, draw the vertical section on cutting plane X-X showing all the necessary constructional details.(name all roof parts)

- The front and rear elevations (scale 1:75).

- Draw using scale 1:10 the plan of course one and two, and the isometric view at a corner.

- At the bottom right hand corner of your answer sheet, draw a title block and in it print your name, registration number, title of drawing, scale and examination paper code.